Boeing Autonomous

Report 2

Elise Chantegros, Budget Liaison

Selina Cozens, Project Manager

Jacob Keeland, Brakes & Manufacturing

Marek Lemieux, CAD Lead

Desirae Mangum, Fundraising Lead

Natasa Peric, Document Editor

Lily Quintanar, Intra-group Communications

Fall 2025-Spring 2026

Project Sponsor: Amanda Nemec (Boeing)

Sponsor Mentors: Donald Guzman and Connor Nolan

Instructor: Professor David Willy

Group Number: F25toSp26_02

# DISCLAIMER

This report was prepared by students as part of a university course requirement.  While considerable effort has been put into the project, it is not the work of licensed engineers and has not undergone the extensive verification that is common in the profession.  The information, data, conclusions, and content of this report should not be relied on or utilized without thorough, independent testing and verification.  University faculty members may have been associated with this project as advisors, sponsors, or course instructors, but as such they are not responsible for the accuracy of results or conclusions.

# EXECUTIVE SUMMARY

This report will discuss the progression and most recent accomplishments of the Boeing Autonomous capstone project. The goal of the project is to develop an actuation and control system that can be retrofitted to any existing vehicle. The vehicle of choice for this project was chosen to be the Coleman GK200 Go Kart. The project has been broken up into three main sub-teams: brakes, steering, and throttle. Additionally, there is an electrical sub-team currently working to create a program to control the kart to function autonomously. This report will focus on the mechanical aspects for each of the three main sub-teams.

In the beginning of this project, the team was given a list of customer and general requirements provided by the clients’, Boeing. From those requirements each sub-team created engineering requirements related to their functions and analyzed these requirements using a House of Quality chart to determine wat requirements needed to be prioritized. Afterwards, benchmarking was conducted for each sub-team to help in determining what goals and standards should. A literature review consisting of multiple sources and mathematical modeling was conducted individually to analyze and predict reactions before prototyping. The concept development process was conducted for each sub-team to select a beginning idea to base their first prototype on. Initial concepts included a booster with solenoid, a DIY EPS kit, and a linear actuator, though several of these since evolved based on updated constraints.

To visualize the team’s current progress and future plans, general Gantt charts for both the fall and the upcoming spring semesters were created. The team plans to complete the mechanical portions of the subsystems before the semester is over. The team has raised $3,473.16 and received a $5,000 sponsorship from Boeing and has estimated a projected total budget is estimated to be around $7,900. Despite funding exceeding projected expenses, the team will continue to fundraise for unexpected costs. For organization, a Bill of Materials (BoM) table was created for each sub-team to keep track of what parts/materials might be needed for the first prototype, along with an estimate of the total amount of money each prototype might cost. Lastly, each sub-team’s Failure Modes and Effects Analysis (FMEA) is discussed; based off the sub-team’s most current BoM.

Prototype 1 was conducted. For all sub teams their main question for Prototype 1 was to determine how to mount their modifications onto the go kart. For the brakes sub-team, their solution was to have the brake caliper mounted under the vehicle in the mudguard area using four L-brackets fixed to the kart with a base plate mounted near the rear left wheel. Space was left for the motor and spool to actuate the caliper lever. For the steering sub-team, they 3D printed CAD models that were designed to hold a motor and the electrical components of the subsystem into place. The 3D printed prototypes had infused C-clamps that were designed to wrap around the poles of the front rack of the go kart and would be fastened into place using bolts and nuts. For the throttle sub-team, they mounted a stepper upside down on the overhanging rack, opposite to the throttle level using brackets. Their assembly included a steel sheet with Styrofoam to dampen vibration. From these prototypes, the sub-teams have analyzed their results and will use their findings to influence Prototype 2, which is currently in progress.

Through the defined requirements provided by the clients, performance metrics, requirement analysis using multiple methods, and updated mathematical modeling, the team has established a strong technical foundation for the next stages of development, which is prototyping. These efforts were made to ensure that the mechanical components of the autonomous’ go-kart’s braking, steering, and throttle subsystems will progress towards a safe, responsive, and reliable operation that could meet both the clients’ expectations and engineering standards.

# TABLE OF CONTENTS

Contents

DISCLAIMER	1

EXECUTIVE SUMMARY	2

TABLE OF CONTENTS	2

1	BACKGROUND	1

1.1	Project Description	1

1.2	Deliverables	2

1.3	Success Metrics	2

2	REQUIREMENTS	Error! Bookmark not defined.

2.1	Customer Requirements (CRs)	3

2.2	Engineering Requirements (ERs)	4

2.3	House of Quality (HoQ)	6

3	Research Within Your Design Space	8

3.1	Benchmarking	8

3.2	Literature Review	9

3.3	Mathematical Modeling	19

4	Design Concepts	23

4.1	Functional Decomposition	23

4.2	Concept Generation	26

4.3	Selection Criteria	31

4.4	Concept Selection	32

5	Schedule and Budget	39

5.1	Schedule	39

5.2	Budget	41

5.3	Bill of Materials (BoM)	41

6	Design Validation and Initial Prototyping	44

6.1	Failure Modes and Effects Analysis (FMEA)	44

6.2	Initial Prototyping	46

6.3	Other Engineering Calculations	48

6.4	Future Testing Potential	51

7	CONCLUSIONS	53

8	REFERENCES	54

9	APPENDICES	60

9.1	Appendix A: Descriptive Title	60

9.2	Appendix B: Descriptive Title	Error! Bookmark not defined.

# BACKGROUND

This chapter of the report introduces the methods of research and ideation that Boeing Autonomous has taken to make the most recent progress with the project. The main sections discussed include the project’s background, requirements, research efforts, design conception, finalization, and sub-team selections. Each sub-team has come up with its own process and thoroughly discusses the parameters of its progress within the respective chapters.

## Project Description

This project will consist of developing an actuation and control system that can be retrofitted onto an existing vehicle, such as a go-kart (see Figure 1 for purchased go-kart). The system will focus on controlling three main functions of the vehicle: brakes, steering, and throttle through hydraulic and/or electronic actuators. The vehicle should be able to function autonomously once these subsystems connect to open-source self-driving software. Its data will be collected through sensors for the system to provide support to the system such as lane-following assistance and obstacle detection.

Figure 1: Vehicle of Choice

Boeing has funded $5,000 for Boeing Autonomies’ project endeavors, such as purchasing the vehicle of choice, safety and protective equipment, and prototypes. According to the projected budget, the team has estimated approximately $800 for brakes, $1,000 for steering, $600 for throttle, $2,800 for electrical, $400 for travel, $1,600 for the go-kart, and $500 for miscellaneous expenses. These values lead to an estimated budget of $6,800, which is below our current funds of $8,473.16. Boeing Autonomous plans to continue to fundraise the rest of the semester through GoFundMe donations and restaurant fundraisers. Currently, $3,473.61 has been raised through GoFundMe, and two restaurant fundraisers have taken place in the past four weeks. With a new fundraising event for a local restaurant next week.

This capstone project can help the advancement of autonomous driving technology for both conventional and unconventional vehicles, such as go-karts and buggies. Through extensive testing and prototyping, the research and development the team has recorded can help accelerate the adoption of autonomous conversion to a variety of vehicles. This advancement can improve accessibility and user experience, along with the reduction of carbon emissions. Additionally, with more efficient and accurate controls, this improved method of driving can reduce redundant maneuvers on the road, such as incorrect turns, and reduce traffic congestion for better traffic flow.

## Deliverables

The deliverables for this project largely center on completed work and presentations for the final prototype of the autonomous vehicle. However, each deliverable is slightly different for the course and client.

The course’s deliverables include a team charter, peer evaluations, an initial design report, as well as a conceptual design report, three presentations, meeting minutes, final CAD and BoM, prototype iterations, and a team website. There are also individual assignments focused on self-learning and engineering analysis. The team charter establishes guidelines for project etiquette so that all members may act accordingly, and the project can run as smoothly as possible. This is shared with the instructor and clients so that all relevant parties are knowledgeable of such information. The peer evaluations are made to ensure that all members are following the team charter, as well as to communicate any issues to the course instructor. The presentations and reports are meant to showcase the progress made throughout the project relating to concept selection, conceptual design, budget, and scheduling. They also include individual research and analysis with the Literature Review and Mathematical Modeling sections, divided by team members. Meeting minutes and agendas are written up as a “Staff Meeting” document to keep meetings organized and to ensure all discussions are recorded for future reference. The final CAD and BoM deliverables act as a final design proposal and encompass all conceptual design work done throughout the semester. The prototype iteration deliverables keep the instructor informed of progress with practical demonstrations to actualize conceptual design work. The team website displays the team and all the work done within the semester.

The client deliverables are similar to their respective course counterparts, as this ensures all relevant parties receive the same information and are able to advise accordingly. The client deliverables are based on reporting progress, by which the presentations, reports, prototype iterations, and the website demonstrate the team’s achievements. Coupled with these deliverables are the team charter, which offers the team’s responsibilities and promises for the duration of the project. Similar to the course guidelines, the presentation and reports showcase conceptual design and intellectual contributions, while the prototype demonstrations are the actualizations of this work. The clients will also check the team website once all deliverables have been added, so that everything is in one place.

This team is not affiliated with any competitions, meaning there are no additional deliverables for such a case.

## Success Metrics

The success of this project largely centers on whether the vehicle can drive autonomously and respond to the control without any safety issues. In terms of the success matrix, the braking, steering, and throttle mechanisms, as well as their interactions, will be assessed along with the electrical components.

For all sub-components, response times will be measured for each mechanism as well as their corresponding time within the neural network. The ideal response time, as well as the required figures for the project to be a success, are defined in Engineering Requirements.

The brake system will work successfully once it is able to safely react according to its surroundings, whether that is coming to a complete stop or regulating the go-kart's speed. It must also be able to transfer control to the user once manual override is enacted. Measurements for the required forces will be applied to regulate speed and ensure that the braking system employs those forces in the appropriate instances. The system's response to certain instances is dependent on the neural network detecting obstacles. The braking mechanisms will successfully respond to the neural network when the neural network can send out the right signal to the mechanisms, which can then transmit the correct force respective to that signal.

For the steering system, the control device must apply the right amount of torque to the wheel for the vehicle to navigate safely and respond to manual override. In terms of torque, the feedback from the torque sensor and steering wheel angle will be compared to calculation-based predictions to examine how the mechanical components will behave. The design must also include a control unit attached to the steering column that includes a kill switch mechanism. To determine that the kill switch is successful, the user interference and the neural network must be easily accessible to remain in charge of the control device. The kill switch must be easy to access and be intuitive for the driver to utilize should they need it. It must also interact with the neural network in such a way that it halts autonomous operations and turns control over to the user. The kill switch metrics will be tested practically in a safe environment once attached to the go-kart.

Like the brake and steering systems, the throttle will be deemed successful once it responds as expected to the control programming in a safe manner. It must achieve a target speed decided by the neural network at a close-to-constant acceleration to avoid jerking. The force enacted onto the system will be determined once the neural network sends a signal and when the vehicle begins to accelerate. Calculations for expected speed and acceleration will be used, given a certain force, to compare the findings to the recorded measurements. The throttle must be able to achieve maximum speed without exceeding it. The throttle must also disengage once the kill switch is used or enough manual override force is enacted.

# Requirements

To ensure the successful progression of the project, it is important to establish and define the requirements from both the clients’ perspective and an engineering perspective. Requirements are an important factor of this project because they provide the team with our constraints, expectations, and goals. In this section, the customer requirements (CR’s) will be discussed for the capstone overall, and the engineering requirements (ERs) will be broken down into sub-teams. To aid in organization, each sub-team has created a QFD (Quality Function Deployment) chart to identify the most important engineering requirements that should be prioritized.

## Customer Requirements (CRs)

This section details the client-specified needs, including autonomous navigation, obstacle detection and avoidance, manual override, real-time state monitoring, safety features, and performance criteria such as speed limits and battery life. Below is a list of the customer requirements given by our clients.

- Autonomous Navigation: The go-kart must autonomously navigate a predefined path from a starting point to a destination without human intervention, following the designated route accurately.

- Obstacle Detection and Avoidance: The go-kart must detect static (i.e., walls, cones) and dynamic (i.e., moving objects) obstacles in its path to adjust its trajectory and avoid collisions safely.

- Manual Override Functionality: The system must allow the user to take control of the go-kart through a mechanical interface, enabling manual operation to override autonomous functions when necessary.

- Real-Time State Monitoring: The go-kart must provide real-time feedback to a user interface, displaying critical system information such as battery life, current speed, and any applicable live video feed to monitor its operational status.

- Emergency Stop Mechanism: The go-kart must include a physical emergency stop button that immediately and safely halts the vehicle.

- Fail-Safe Design: The system must stop safely in the event of a critical component failure, ensuring no unsafe operation occurs.

- Battery Life for Continuous Operation: The go-kart must operate autonomously for at least 20 minutes on a single battery charge to support continuous operation during demonstrations or testing.

- Support for Driver Assistance and Autonomous Modes: The control system must support both driver assistance features (e.g., lane-keeping assist) and fully autonomous modes, allowing flexibility in operation.

- Actuator Control for Steering, Braking, and Throttle: The system must enable lane-keeping assist and other driver aids by controlling the go-kart’s steering, braking, and throttle, ensuring smooth operation in both autonomous and driver-assisted modes.

- Integration with Open-Source Autonomous Platforms: The system must integrate with open-source self-driving software and sensor data to enable autonomous driving capabilities.

- Prioritized Safety Features: The system must prioritize safety features like emergency stop, watchdog timers, and validated fail-safe mechanisms to ensure safe operation at all times.

- Closed-Course Testing with Controlled Parameters: The control system must be tested on a closed course with defined performance parameters (e.g., speed, path accuracy, obstacle avoidance) to validate functionality and reliability.

- Comprehensive Documentation and Handover: The project must deliver a complete handover package, including a System Requirements Document (SRD), Requirements Traceability Matrix (RTM), Interface Control Document (ICD), CAD models, Bill of Materials (BOM), software integration details, test reports, a prototype demonstration video, and an operator’s manual detailing how to use the self-driving features.

## Engineering Requirements (ERs)

This section summarizes quantifiable performance, safety, reliability, usability, scalability, and maintainability requirements provided by the client, along with additional engineering specifications defined to meet client expectations. The client has provided the following list of requirements to ensure the successful completion of the go-kart’s autonomous construction.

### General Engineering Requirements

The system's obstacle detection and avoidance algorithms shall have a response time of less than 500ms to ensure safe operation. The buggy's top autonomous speed shall be no more than 15 kph (9.5 mph). The system shall operate for a minimum of 20 minutes on a single battery charge during continuous autonomous operation.

### Brakes Engineering Requirements

- Emergency Stop Time:  2s

- Emergency Stop Distance ≤ 5m

- Lifespan: 10hrs

- User Brake Input Force ≤ 2.5N

- Brake Deceleration ≥ 7𝑚/𝑠^2

- Automatic Brake System Operable for ≥ 20 minutes

### Steering Engineering Requirements

- Minimum Turning Radius (7m)

- Max Rotation Range (900°)

- Minimum Autonomous Torque (2.5Nm)

- Minimum User Torque (2Nm)

- Max Autonomous Turning Velocity (10kph)

- Max Autonomous Turning Acceleration (15km/(h*s))

- Response time (< 500ms)

### Throttle Engineering Requirements

Specifications for the throttle have been altered for a more accurate approach to the engineering requirements. Based on iterated research and calculations, the updated list represents the safe range of values for each criterion in terms of how the throttle must function.

- Maximum Speed: 15mph

- Power Consumption: <100W

- Manual Override Response Time: <500ms

- Actuation Response Time: 37-54ms

- Pedal Force Range: 19-29N

- Manual Override Force Range: 19-22N

- Throttle Cable Velocity Range: 0.46-0.66m/s

## House of Quality (HoQ)

The House of Quality (HoQ) is one of the matrices of the Quality Function Deployment. The HoQ is utilized to maximize efficient planning that considers the clients’ needs and requirements, the team’s understanding of its clients, and the technical engineering requirements for the project. Boeing Autonomous has created a HoQ for each sub-team for focused decision-making.

### Brakes

The House of Quality (HoQ) presented for the autonomous go-kart's braking subsystem serves as a critical tool to align the customer requirements with the technical specifications, ensuring a robust and effective design process. The team has meticulously defined customer needs, such as reliable braking performance, rapid response times, and safe operation under varying conditions, as outlined in the customer requirements document (e.g., for the emergency stop mechanism and for fast response to obstacles). These needs are mapped against technical requirements such as braking force, actuation time, and power consumption derived from the System Requirements Specification and project kickoff details. The HoQ, with its weighted relationships and prioritized targets (e.g., braking within 500ms as per SRS 3-1), allows the team to identify key areas of focus, such as optimizing brake actuator responsiveness and ensuring durability. By integrating customer feedback, like the emphasis on safety from the Canvas project file, and balancing trade-offs (e.g., power consumption vs. performance), the group aims to deliver a braking system that not only meets but exceeds expectations.

Figure 2: Brakes QFD

### Steering

The HoQ for steering displays the customer needs and weights that were derived from a list of functional and non-functional requirements given by the clients. For the customer needs, the steering sub-team is focused on autonomous navigation, obstacle detection, user control, obstacle avoidance, safety, performance, and usability. The customers’ needs were compared with three companies chosen for benchmarking, Tesla, Waymo, and Cruise. The technical requirements are response time, minimum turning radius, maximum rotation, minimum autonomous torque, minimum user torque, maximum autonomous turning velocity, and maximum turning acceleration. As depicted in Figure 2: Steering QFD below, response time is the most important technical requirement based on the weighted customer needs.

Figure 3: Steering QFD

### Throttle

The QFD for the throttle subsystem followed similar criteria to those for the brakes and steering systems. However, there have been new changes in its technical requirements because of new parameters used to define the success of the throttle mechanism. Because its success mainly depends on pedal force and manual override force, changes to the technical values were crucial in accurately weighing out the

importance of each technical parameter. Other technical requirements included actuation response time, manual override response time, cable velocity, maximum speed, and power consumption. These were then compared to customer needs such as autonomous navigation, user control/manual override, state monitoring/diagnostics, performance, safety, and usability. The customer surveys from Tesla, Waymo, and Cruise helped compare these needs. Through the QFD process, pedal force and manual override force were the most relevant criteria for designing the throttle mechanism. The sub-team concluded that pedal force significantly impacts the go-kart’s safety and efficiency.

Figure 4: Throttle QFD

# Research Within Your Design Space

In this project, the effort of the team is represented in understanding and analyzing existing technologies, relevant literature and patents, and mathematical analyses that have informed the development and design of each sub-function at its current state. Subsystem-level benchmarking was conducted to identify targets and design insights for the project. Afterwards, an extensive literature review was conducted, individually summarizing multiple sources that have provided insight and guidance. Finally, mathematical modeling was used to solve important inquiries that could affect the design and enabled the team to predict performance, optimize designs, and ensure compatibility across the project.

## Benchmarking

To generate an idea of the current state of the field of automated vehicles, the top autonomation companies, Tesla, Waymo, and Cruise, were evaluated. Each company highlights important areas of consideration, such as whether the vehicle should steer using lidar or cameras or both. Waymo has gained traction for the issues with training a lidar system. Tesla uses highly specified cameras in which they compare their system to human eyes [1], while Waymo and Cruise have LiDAR control systems [2] [3]. Nonetheless, these differences still allow both companies to produce the best results. LiDAR cameras have strong depth perception being able to detect within 30cm-3mm. However, this clarity does not remain as accurate for objects at a distance greater than 200m. Cameras can detect objects within 1.2 megapixels but the disadvantage is a lack of accurate depth perception and computation power. Using these companies as benchmarks will allow us to consider reoccurring challenges within the design space.

## Literature Review

### Elise Chantegros

A Hybrid Model for Vehicle Acceleration Predictions [4]

This paper uses real driving data, clusters different acceleration patterns, and then applies deep learning to more accurately predict a vehicle’s short-term acceleration. For our project, this idea is relevant because better acceleration prediction can help the controller choose safer, smoother throttle commands instead of just reacting in real time.

Design and Implementation of a Fault-Tolerant Drive-by-Wire System [5]

This thesis develops a fault-tolerant drive-by-wire system that uses redundancy, failure detection, and safe-state responses to maintain control of steering, braking, and throttle even when components fail. It demonstrates how distributed control, hardware monitoring, and safety-oriented structures improve the reliability of electronic vehicle systems. This relates to our project because it involves design principles such as redundancy, component monitoring, and fail-safe behavior, which are important when adding autonomous features to any vehicle system, including throttle control.

Electronic Throttle Control – A Dependable Case Study [6]

This paper evaluates the safety and reliability of electronic throttle control systems by modeling faults, redundancy, and ECU monitoring strategies to prevent unsafe events like unintended acceleration. The authors compare multiple ETC architectures and show how added checks and redundancy greatly improve safety with minimal extra cost. This relates to our project because it highlights proven safety practices, like sensor redundancy, plausibility checks, and fail-safe modes, that can inform how we design a safer autonomous throttle system.

Model Predictive Path Following Control for Autonomous Cars Considering a Measurable Disturbance: Implementation, Testing, and Verification [7]

This paper presents a model-predicative path-following controller for autonomous cars that accounts for road boundaries, vehicle shape, and disturbances from changing road conditions. To make the controller fast enough for real-time use, the authors replace traditional solvers with a differential-evolution-based optimization method and experimentally verify that it performs reliably on an autonomous test vehicle. This relates to our project because it highlights how predictive control and real-time optimization can improve autonomous steering and navigation.

Accurate and Smooth Speed Control for an Autonomous Vehicle [8]

This paper presents a preview-based control algorithm for automated vehicles that uses future road slopes and target speed information to improve speed tracking. Compared to a standard PID controller, it achieves smoother throttle and braking, lowers tracking errors, and improves ride comfort. This method is relevant to our project because it demonstrates how incorporating predictive information into control systems can improve performance and vehicle response. Implementing a similar approach could help optimize our throttle control for a more precise and efficient operation.

Throttle Control of Engine by Using PMDC Motor and Drive [9]

This paper discusses the design and control of an electronic throttle control (ETC) system using a permanent magnet DC (PMDC) motor, which allows precise regulation of the engine airflow through feedback and pulse-width modulation (PWM). It highlights how the system achieves fast and accurate throttle responses using PID and nonlinear control strategies, despite nonlinearities and friction. In our go-kart, this system can automatically manage the throttle position to control speed and acceleration safely and efficiently.

Diagnosis of Automotive Electronic Throttle Control Systems [10]

This article discusses how drive-by-wire (DBW) systems replace traditional mechanical linkages with electronic controls, which allow the driver's inputs, such as throttle, brake, and steering, to be managed through sensors and actuators. It demonstrates that Electronic Throttle Control (ETC) is a key subsystem of DBW, where the accelerator pedal sends signals to the ECU, which then adjusts the throttle actuator electronically. This setup allows for smoother and more precise throttle modulation without having mechanical cables. This paper relates to our project as integrating ETC provides for the computer to control acceleration through electronic signals instead of manual input autonomously.

Drive-by-wire Conversion for Autonomous Vehicle [11]

This article discusses drive-by-wire (DBW) technology, which replaces mechanical linkages with electronic systems such as throttle-by-wire, brake-by-wire, and steering-by-wire. DBW has benefits such as reduced weight and improved precision, however, it faces challenges with software reliability and sensor failure. This technology is relevant to our project as it enables autonomous control of the throttle and pedals through software commands.

Digital Twin System for Automotive Safety with Integration of a Safety Controller [12]

This thesis presents the development of a Digital Twin System for autonomous vehicle applications, integrating real and virtual environments to test and validate control systems in real time. It focuses on the modeling and simulation of vehicle dynamics, actuator control, and electronic subsystems such as throttle, brake, and steering within a hardware-in-the-loop framework. By enabling safe and efficient testing without requiring physical prototypes, the Digital Twin System supports the design and refinement of autonomous driving functions. This is relevant to our project because it demonstrates how throttle and other drive-by-wire systems can be virtually modeled and controlled to optimize autonomous performance and system integration.

Autonomous Braking and Throttle System: A Deep Reinforcement Learning Approach for Naturalistic Driving [13]

This paper represents a Deep Reinforcement Learning approach using the Deep Deterministic Policy Gradient algorithm to create an autonomous braking and throttle system that mimics human driving. The system was tested in simulations to smoothly adjust throttle and brake inputs for collision avoidance and natural vehicle control. This is relevant to our throttle subsystem because it demonstrates how learning-based control can enable smooth, adaptive throttle response for autonomous operation.

### Selina Cozens

Reinventing the Wheel: Transforming Steering Wheel Systems for Autonomous Vehicles [14]

This article conducts a study on two different types of steering wheels: robotic and LED. These steering wheels were programmed to warn the driver of upcoming hazards. The robotic steering wheel warned the driver mechanically, while the LED steering wheel warned the driver physically. The robotic steering wheel was found to be more effective at getting the driver’s attention. This study may be utilized as a potential safety modification for the project.

Nonlinear Steering Wheel Angle Control Using Self-Aligning Torque with Torque and Angle Sensors for Electrical Power Steering of Lateral Control System in Autonomous Vehicles [15]

Most modern-day cars use electric power steering (EPS). However, for the EPS to work properly, the self-aligning torque is seen as a disturbance and is rejected. However, this study proposes an idea to utilize this self-aligning torque through a nonlinear control of the steering wheel angle. This study can be useful for the design of this project's DIY EPS kit.

Electric Power Steering Automation for Autonomous Driving [16]

This paper discusses a fuzzy-based control system that can be applied to mass-produced electric power steering systems that mimic human behavior. The control system of choice for the study was the Citroën C3 Pluriel. Some experiments were conducted on a private test circuit, which was found to obtain human-like behavior in all mannerisms. This study can help the steering sub-team with programming the EPS.

Predictive Active Steering Control for Autonomous Vehicle Systems [17]

In this paper, a study is performed using a model predictive control (MPC) to control the steering wheel for autonomous vehicles. The MPC is used to calculate the steering angle required to navigate through icy or slippery roads. There are two approaches this test takes, a nonlinear vehicle model and a successive online linearization of the vehicle model. The study can be useful for the programming of the steering sub-team's EPS, since Flagstaff is a snowy city.

Automatic Steering System [18]

This paper discusses the creation of a “pre-notification” that can be attached to both automatic and manual vehicles to reduce the chances of car accidents and further advance the creation of autonomous vehicles. This safety modification consists of having the motor of the EPS connected to an electrical control unit (ECU) and moving with the steering shaft in both manual and automatic steering modes. This study can be useful when it comes to the design of the DIY EPS kit.

Real Time Ackerman Steering Angle Control for Self-Driving Car Autonomous Navigation [19]

The Ackerman Steering Angle is a well-known formula that can predict the angles that a vehicle can make based on its dimensions. This study proposes an idea for autonomous steering to use this formula with sensors that are used for lane line detection. The use of sensors and the Ackerman Steering Angle is very similar to the draft idea the sub team has for the DIY EPS kit. This study can be useful for implementing the Ackerman Steering Angle formula into the controlling unit.

Steering Wheel Systems (393.209) [20]

According to “The Motor Carrier Safety Planner” in section 5.1.12 Steering Wheel Systems (393.209), steering wheels must be securely and properly attached to the automotive securely and properly without any spokes cracked or missing. For power steering systems, all components must be in operating condition. No parts shall be broken or loose; belts shall be in good condition. There should also be no leaks, and the system should have sufficient fluid in the reservoir. These standards will aid the steering sub-team by making sure that the steering wheel and its power steering component are safe according to these standards.

Investigation on 3D Printing Parameters of PLA Polymers for Gear Applications [21]

Fused Deposition Modeling (FDM) enables low-cost complex part fabrication with a wide range of materials, making it a popular option across industries. This study experiments with how various FDM parameters, such as temperature, layer height, print speed, and infill pattern, affect the mechanical and tribological (wear and friction) behavior of PLA for gear applications. This study will aid the steering sub-team by providing more insight for using 3D printed PLA for prototyping a helical gear train.

Feasibility Investigation of Gears Manufactured by Fused Filament Fabrication [22]

This paper experimented with the production and testing of plastic gears made by fused filament fabrication (FFF). Different 3D printers of high and low quality, along with multiple materials, were used to print the gears. The study found that the quality and accuracy of the geometry from the printing process were not responsible for the failure of the gears. Instead, for PLA specifically, the gears failed due to the tooth root heating up and affecting the material properties. This study will help the steering sub-team prevent 3D printed prototype gears from overheating and failing.

Surface durability of 3D-printed polymer gears [23]

This study investigates the feasibility of using 3D-printed gears in unlubricated mechanisms. Many types of filaments were tested, and factors were analyzed, such as sliding speed, material hardness, surface finish, geometry, and microstructure influence wear. The study found that PLA demonstrated better abrasive-wear resistance than ABS, but ABS had superior overall structural integrity. The findings in this study can influence the type of material that can be used for prototyping the steering prototype helical gear train and can guide evaluation performance and wear and tear behavior.

### Jacob Keeland

Race Car Vehicle Dynamics [24]

The definitive reference book on vehicle dynamics specifically tailored to race cars, covering tire behavior, suspension kinematics, aerodynamics, and full-vehicle handling.

Shigley’s Mechanical Engineering Design [25]

Standard mechanical engineering textbook covering machine design principles, including stress analysis, failure prevention, bearings, gears, and fasteners.

Brake Design and Safety [26]

Comprehensive guide to automotive brake system design, performance calculation, and safety standards, widely used in both OEM and motorsport brake engineering.

350 kg·cm High-Torque Digital Servo Datasheet [27]

Official datasheet for an ultra-high-torque (350 kg*cm at 8.4 V) digital servo commonly used in large-scale robotics and heavy-duty RC applications.

Physics of Braking Dynamics [28]
	This article by Halliday and Resnick in Fundamentals of Physics examines the fundamental principles of force, motion, and energy dissipation. The research provides a theoretical foundation for understanding braking dynamics, which could optimize the go-kart’s stopping performance to meet.

Motor Driver Control Techniques [29]
	This Amazon study explores the DEVMO BTS7960B motor driver module, investigating PWM control techniques and current handling up to 43A. The research offers insights into precise braking actuator management, which could enhance the go-kart’s safety and responsiveness as required.

Solenoid Valve Endurance Evaluation [30]
	This eBay article focuses on endurance testing and failure modes of 12V DC solenoid valves, yielding insights into long-term reliability. This research could guide the development of a robust braking system capable of withstanding the repeated testing cycles outlined in.

High-Performance Valve Dynamics Study [31]
	This article from HydraForce investigates the dynamic response and fluid control of the 12V DC NC solenoid valve SV10-22-0-N-00-9-5112-NC under varying loads. The research provides valuable data for optimizing braking performance during emergency conditions, supporting the design goals.

Solenoid Valve Operational Dynamics [32]
	This article from eBay examines the operational dynamics of 12V DC solenoid valves, providing technical data on pressure thresholds and cycle life. The research offers insights into their reliability for rapid braking responses.

Kuiper Collection Mechanical Component Investigation [33]
	This article explores the Kuiper Collection’s research into mechanical component designs, offering insights into lightweight materials and historical braking mechanisms. The study provides data on material durability that could inspire innovative approaches to the go-kart’s braking system.

12V DC Solenoid Valve Performance Analysis [34]
	This article conducts a study on the performance of 12V DC solenoid valves, focusing on flow rates and response times under various operating conditions. The research highlights how these valves respond to rapid actuation, providing critical data on their potential to enhance braking efficiency and reliability in autonomous systems. This study may be utilized to inform the design of a responsive braking system for the go-kart, ensuring it meets the fast response requirement of 500 milliseconds.

### Marek Lemieux

Vehicle Braking, Stability and Control [35]

This analysis studies the braking systems and tires of a vehicle set to brake in a straight line compared to a control that drives without using brakes. Comparing the measurements found in the study to those gathered through group testing could provide information on critical events such as rollovers, accidents, and how steering affects the braking system.

The Effect of Tire Break-In on Force and Moment Properties [36]

The article studies the interactions between the tires of a braking vehicle and the road beneath it. Importantly, the article notes that there is no possible way to have a single governing equation to study the effects of braking tires on the surface. In the future of this project, special modeling considerations will be needed for the braking system based on the curvature of the road, the road consistency, the weather conditions, and any other important outside factors.

Analysis of heat conduction in a disk brake system [37]

The troubles outlined in this study concern the possibility of brake fluid vaporization caused by frequent or long-distance disk braking. Though this should not be of concern for short-term applications, the consistent use over the preceding testing cycles could have a significant impact on brake fluid, impacting results. Another use of this article is to use the force applied to the disk to calculate energy loss from the system.

On the Distribution of Braking Forces in Road Braking [38]

Using road regulations from Finland this paper studies the effects of braking forces on a vehicle’s axels. This can be used with other sources for modeling to predict the maximum turning forces applied to the vehicle while braking that can be accomplished without losing brake effectiveness.

Acceleration-Deceleration Behavior of Various Vehicle Types [39]

This paper analyses the behaviors of drivers of different sized vehicles when approaching or leaving a stop light. These statistics can be used to influence where stop lights are placed, but they also include determination of sight and statistics for human drivers. With the data measured, comparisons can be made between human drivers and the autonomous system being developed.

Autonomous Vehicles Testing and Operating in the State of Arizona [40]

These are the current Arizona laws of autonomous vehicles operating on roads and streets. In this stage, it is almost certain the vehicle will not be testing on public roads, but it is still good to have every case covered in case further testing is needed.

Braking distance [41]

This article explains braking distance history, uses, and simple formulas. Using the simple models in this article will be a very rough estimate of the karts braking distance. Further use of modifying these equations could yield more precise estimates in future testing.

Acceleration [42]

This article explains acceleration history, uses, and simple formulas. Using the simple models in this article will be a very rough estimate yielding the vehicles' deceleration from maximum speed. Further use of modifying these equations could yield more precise estimates with further testing.

Design and analysis of a brake caliper [43]

This paper focuses on analyzing a typical brake caliper and the forces applied using some very interesting finite analysis. There are plenty of usable formulas and constants found within the report that will make future analysis more accurate.

Motor Torque Calculations for Electric Vehicle [44]

This article is a much more generalized look at the torque found within electric vehicles in several states. Several related formulas and a table of rolling resistance coefficients are particularly helpful in analysis.

### Desirae Mangum

Steering System and Method for Autonomous Vehicles [45]

This resource is a European patent specification for an autonomous steering mechanism, including methods for operation. This document will allow us to generate a well-rounded idea of what we need to design moving forward. The description of how it works provides a deeper understanding and insight into what we are designing to be able to do.

Intelligent Electric Power Steering: Artificial Intelligence Integration Enhances Vehicle Safety Performance [46]

This article focuses on electric power steering systems and how they have developed over the years. Discussing the safety parameters surrounding power steering systems and the challenges that occur when AI is implemented into the automotive field. Using this knowledge to not only understand how these systems work, but also how we can use them to our advantage while designing our own system.

Analysis and Robust Control Design of Steering System for Autonomous Vehicles [47]

This resource provided an analysis of the steering systems for autonomous vehicles with a robust control design. Showing both the simulation and the methods applied to perform the required calculations. This article provided a unique perspective on the results produced, allowing a deeper understanding of autonomous vehicles and what knowledge is needed to be successful.

Simulation Studies – Path Tracking of an Autonomous Electric Vehicle (AEV) by Using Fuzzy Information of Speed and Steering Angle [48]

This article discusses the issues that will arise when you are turning a vehicle autonomous, more specifically, electric vehicles. Knowing potential issues will allow us to plan to avoid these issues or design to solve these problems. This knowledge will allow us to stay ahead of the problems and stay on our timeline to have a successful project.

Model Predictive Control-Based Path Tracking with Four-Wheel Independent Steering, Driving, and Braking Autonomous Vehicles on Low Friction Road [49]

This report focuses on a similar vehicle setup to what we will be working on, and the controlling aspects to consider. Providing insight into how to create a predictive control system based on path tracking, touching base on the braking system to ensure that all safety components are being taken into consideration. The takeaway from this source is that a model predictive control system requires multiple trials to ensure that all systems are properly executed.

Development of Steering Control System for Autonomous Vehicle [50]

This report is a similar study to that of what we will be producing by the end of this project, focusing on the steering of autonomous vehicles. Providing a background of steering technology and an example method of integrating an autonomous system into existing systems. Using this article to have an idea of the direction of the project, gaining ideas for calculations and potential results.

What is the Role of Steering Angle Sensor in Autonomous Driving? [51]

This YouTube video provides a brief explanation of the roles of the steering angle for the sensor when implementing an automated system. It explains what exactly the steering angle is and how it can be found and then explaining the importance of the angle when using sensors. This information was used to develop a further understanding of the aspects involved in designing an autonomous steering system.

Automated Vehicles for Safety [52]

This source is from the Department of Transportation, referring to the regulations and safety requirements for autonomous vehicles. Providing a timeline of regulations as they become enforceable, such as antilock brakes or lane-keeping assistance. Along with examples of the different levels of automation at work and the technologies involved. This source was used to understand the safety behind autonomous driving, and a bonus description of the varying levels of automated vehicles.

Machinery’s handbook [53]

This source is a textbook that provides deeper explanations of designing various mechanical components. This source was used to help justify a 1:1 gear ration to show was it could be more beneficial for the team’s design. With various components within the book this source will continue to be useful as the team runs into mechanical errors and issues.

Helical Gear Design in SolidWorks [54]

This video comes from YouTube and allows an understanding of creating size varying helical gears. This source was used to show how gear can easily be changed when equations are used for the dimensions. With this new skill iterations of gears will be easier to produce within SolidWorks, allowing for more time to focus on mechanical issues within prototyping.

### Natasa Peric

A Recommended New Approach on Motorization Ratio Calculations of Stepper Motors [55]

This article defines the different stepper motors that are used in a variety of fields based on their torque and cycle ratios. Because they can be used in a variety of environments such as aeronautical and automotive fields, the article highlights the significance of accurately calculated torque ratios in stepper motors. It also emphasizes the impact of different torques in a mechanism, which has proven to be useful for calculating the net torque in the stepper motor used by the throttle sub team.

3D Printing of Automobile Power Transmission System Using Tough PLA [56]

This paper explains the benefits and possible risks that arise with the use of PLA material in 3D printed components used in the automotive industry. This article is important to the throttle sub team’s approach in 3D printing the spool attached to the motor stepper, as it can be a possible risk over time based on regular use. This paper, thus, highlights the importance of using the correct plastic material to ensure that 3D printed objects can be used for automotive based components.

Past and Future Practical Solutions for Torsional Vibration Damping in Vehicle Industry [57]

This article explains the alternative routes available in overcoming the torsional vibration when a vehicle is in motion through damping mechanisms. It introduces torsional resonance due to the vibrations in an engine that can lead to possible damages and issues within a vehicle. Hence, this article highlights possible risks the added actuation systems may undergo after long periods of drive time when the vehicle is being used.

AUTOSAR-enabled throttle position sensor testing and analysis using MATLAB [58]

This article describes the way MATLAB can be employed to determine and test the parameters of sensors used for a throttle body. It offers an innovative approach to discovering the most effective methods that sensors can be added to a throttle mechanism through an effective simulation. This feature will be important in exploring once sensors are chosen for the throttle components and how they can be accurately monitored through MATLAB.

Multi-Parametric Analysis of a Mimicked Accelerating Pedal (Via DC Motor) of an Electric Vehicle [59]

This paper details the use and abilities of throttle pedal when it is controlled by a DC motor for an electric vehicle. Because it employs a DC motor for its mode of control, the paper offers important details to actuating the pedal, such as the different torque and distances crucial to the implementation and calculations that will allow the pedal to be controlled. This information is important because the throttle mechanism has a very similar approach to actuating the throttle body to make it autonomous.

Safety of 3D Printing in Automotive Industries [60]

This document defines how 3D printed components can be safely added into the creation and improvement of automotive-based machines. It explains the benefits and risks of using different materials such as Nylon 6 compared to traditional materials like aluminum. This evaluation will help the throttle team in deciding what material will be best for the spool design on the motor in the iteration process.

Development of an Emergency Control Algorithm for a Fail-Safe System in Automated Driving Vehicles [61]

This article explains the various options that can be used as a failsafe mechanism for a self-driving vehicle. It outlines the current mechanisms and software systems that are actively used for autonomous vehicles, which can be applicable the fail-safe design built for the throttle sub-system. The NHSTA has proposed several ideas for a fail-safe to be used in autonomous driving, in which the paper offers another approach that may be more universal to the actuation fail-safe for the go-kart. This article, hence, serves as a helpful source in catching the possible risks that must be considered when designing an effective failsafe that must always be employable, especially during operation.

Power Consumption on Arduino Boards [62]

This paper defines how power consumption fluctuates for Arduino-based products and what devices they are compatible with. The throttle body plans to use an Arduino microcontroller, such as a Nano or Raspberry Pi, in which the power consumption will be significant to the control over the throttle’s mechanism. Hence, this paper is important in defining what parameters must be considered for the power source in the throttle’s actuation system.

NHTSA Rulemakings Related to Automated Driving System-Equipped Vehicles NHTSA Rulemakings Related to Automated Driving System- Equipped Vehicles [63]

This document is administered by the National Highway Traffic Safety Administration, and it offers the most recent set of laws and regulations on autonomous vehicles being on public roads. It also involves information from the Federal Motor Vehicle Safety Standards that companies must follow for safe autonomous vehicles (i.e., speed, response, and detection abilities). Hence, this document emphasizes the safety laws and regulations that apply for the go-kart to be considered safe and autonomous by federal safety standards.

Organization of machine learning based product development as per ISO 26262 and ISO/PAS 21448 [64]

This article outlines the ISO standards for machine-learning based algorithms used based on new technology that can apply for electric vehicles. The article offers a list of technical specifications and processing systems that are related to ISO standards to define and limit hazards that can arise in complex technology. It is sourced as an instructive guide for the methods of controls considered to be implemented in the construction of the go-kart’s autonomous abilities, as well as where the possible downfalls lie.

### Lily Quintanar

Research on the Application of Multi-Sensor Fusion in Autonomous Driving [65]

This source explores the implementation of LiDAR, millimeter wave radar, ultrasonic radar, and cameras in autonomous vehicles. It both compares their performance on an individual level and describes the decision logic when used together. This source was important in contributing to the development of our sensor system.

LiDAR-Based Obstacle Avoidance with Autonomous Vehicles: A Comprehensive Review [66]

This source explores the efficacy of the use of LiDAR in autonomous vehicles by itself and with other sensor types. The authors specifically apply this research to electric power steering (EPS) models using an artificial neural network. This source is relevant to the project because we will be utilizing LiDAR sensors, EPS, and a neural network.

Nonlinear Hybrid Impedance Control for Steering Control of Rack-Mounted Electric Power Steering in Autonomous Vehicles [67]

This source proposes a hybrid impedance control system to create a smoother transfer of control of the wheel from the automated system to a driver. This source, while insightful, focuses on a rack-mounted steering system, which we have ultimately decided against.

Autonomous Driving by Using Convolutional Neural Network [68]

This source explains the relationship between three front-mounted cameras, a convolutional neural network (CNN), and the steering wheel. It describes the testing process for obtaining an accurate wheel angle from data collected by the cameras. This source is relevant to our case because it explains a possible way to train a neural network.

Neural Network-Aided Fast Iterative Model Predictive Control for Autonomous Vehicle Motion Control [69]

This source proposes a data-physics system utilizing a long short-term memory neural network to capture data from and predict non-linear aspects of a vehicle, such as road friction variances. This source is incredibly detailed and will be useful when building our convolutional neural network.

Reinforced Curriculum Learning for Autonomous Driving in Carla [70]

The authors of this paper seek to create and utilize a reinforcement learning (RL) system for autonomous vehicles. They test their code in CARLA, a simulator for testing autonomous driving systems. The code ended up having a few issues including low speed and some collisions, however it will be helpful while we train our RL.

Fault Diagnosis Strategy of Autonomous Steering System for Intelligent Vehicle Based on State Estimation [71]

This source utilizes rack displacement as a state feedback signal into its steering system to determine whether the system is working properly or not. This will be useful later when testing and seeking to reduce error.

Training of a Convolutional Neural Network and Its Object Recognition Ability Depending on Illumination and Contrast of Images [72]

This paper concludes that a qualitative data set is necessary to train a neural network for object recognition. This reduces errors with variances in light sources and contrast.

An Improved Neural Network Algorithm for Intelligent Object Recognition [73]

The research of this paper aims to allow for a neural network to recognize objects that may be out of frame or askew. This helps in building the neural network’s ability to recognize any object and act accordingly.

Advancing Object Detection: A Narrative Review of Evolving Techniques and Their Navigation Applications [74]

This source outlines current algorithms for dynamic object detection and their current and future applications within autonomous navigation. This is important to meet Boeing’s object detection requirements.

## Mathematical Modeling

This section presents the mathematical analyses conducted to support the design of the autonomous go-kart’s brakes, steering, and throttle sub-systems. By quantifying key performance parameters such as braking forces, steering torque, and throttle actuation, these models ensure compliance with critical customer and engineering requirements, including rapid response times, safe operation, and precise control. Each sub-team’s analysis provides a foundation for optimizing subsystem designs, validating performance through calculations, and guiding the development of prototypes to meet the project’s autonomous navigation and safety goals.

### Brakes Sub-Assembly – Jacob Keeland and Marek Lemieux

Safety is a key characteristic within the brakes sub-system, along with maximizing the kart’s autonomous capabilities. For ensuring this safety, calculations were performed for the time needed to bring the vehicle to a stop from a given maximum operating speed, as well as the distance traveled in the time it takes for the vehicle to stop. Additional calculations were performed, braking dynamics, weight transfer, required clamping force, cable tension, and actuator torque yielded a substantial factor of safety on both torque and actuation speed, ensuring reliable emergency stops even under worst-case conditions. These values are ultimately used to meet the client’s requirements while still maintaining safety for those around the vehicle during autonomy.

Calculations - Jacob Keeland:

Variables

- Cable tension T=  223N

- Spool Radius (rs)= 0.025m

- Efficiency (η)=0.094

- Required Servo Torque(τreq)=5.93Nm

- Servo’s Max Torque (τmax)= 35Nm

- Factor of Safety (Factorsafety)= 5.90

Equations and Mathematical Computation:

τreq=5.93Nm

τreq=T*rsη

τreq=223*0.0250.094

[75 JK1]

Factorsafety= 5.90

Factorsafety= 355.93

Factorsafety= τmaxτreq

[76 JK2]

Results and Conclusion:

A single Docyke 350 kg*cm servo pulls a cable to clamp a mechanical caliper, locking the rear wheel of our 233kg go-kart. Math proves the motor only needs 5.9Nm and 72ms, yet the servo delivers 35 Nm in under 80ms, giving us massive safety margins and zero need for design changes.

Calculation - Marek Lemieux:

Variables:

- Maximum autonomous speed ( v ) = 4.167 m/s

- Brake efficiency (μ ) = 0.7

- Acceleration due to Gravity (g ) = 9.81ms2

- Reaction time of vehicle (t ) = 0.5 s

- Acceleration of vehicle (a ) = 6.87ms2

- Time to bring vehicle to complete stop (T ) = 1.11 s

- Distance traveled while stopping (D ) = 3.35 m

Equations:

T=t+∆va            a=μg        T = 1.11 s

D=vt+v22a

D = 3.35 m

Results and Conclusion:

Moving forward with these calculations as a baseline, it appears the kart will be able to stop safely, provided there are no catastrophic failures. One factor that will likely improve is the reaction time to begin deceleration. As physical prototypes are built, it is feasible that quicker response times will improve both time and distance to stop. In future models, application of test data will also yield more accurate results and reflect the conditions applicable to reality.

### Steering Sub-Assembly – Selina Cozens and Desirae Mangum

Calculation – Selina Cozens:

The goal for this mathematical analysis is to determine the minimum motor power needed for an electric power steering system, considering a gear train between the motor and the steering rod.​ As represented by the equations below, the minimum motor power required for the steering prototype motor is 25.1W.

Variables:

- Kart Width (KW) = 1.12m [77]

- Wheel Width (WW) = 0.15m [78]

- Minimum Turning Radius (R) = 2m

- Wheelbase (WB) = 1.26m

- Steering Wheel Rotation (SWR) = 3π2 rad

- Steering Wheel Radius (r) = 0.15m

- Force Average (F) = 73.4N

- Safety Factor (Csafety) = 1.4

- Testing Steering Angle (θtest) = π2 rad

- Maneuver Time (tmaneuver) = 0.5s

Equations [79][80][81][82][83][84]:

Track WidthTW=KW-2WW2=0.97m

γavg=tan-1WBR-TW2+tan-1WBR+TW2 /2=0.58rad

Steering RatioSR=SWRγavg=8.10

τwheel=CsafetyrFsinθtest=15.4Nm

τmotor=CsafetyτwheelSR=2.66Nm

ω=SWRtmaneuver=9.42rad/s

Pmotor=τmotorω=25.1W

Results and Conclusion:

Moving forward, the steering sub team should look for a steering column motor that is around 25.1W minimum. Additionally, measurements will be continuously collected in the future that may change these results. Values that were found during this mathematical analysis, such as the torque of the wheel, will be used for future analyses concerning the sub-team’s prototype.

Calculation – Desirae Mangum:

The goal of this mathematical model is to determine the correct gearbox ratio, G, with a set motor torque, TMotor, needed to deliver the torque assist required for our vehicle to drive fully autonomously.

Variables:

- Estimated maximum required torque at steering shaft (TReq,Shaft) = 15.4Nm

- Estimated Motor Torque (TMotor)= 2.66Nm

- Desired assist fraction (a ) = 100% [52]

- Gear Efficiency (ηg) = 0.9

Equations:

Tassist=TReq,Shaft*a               TAssist=15.4 Nm

G=TAssistTMotor*ηg                  G=7:1				[85]

Results and Conclusion:

As represented by the equations below, the torque assist required was found to be equal to the required torque within the shaft with an estimated value of 15.4Nm. With this value, the gearbox ratio was calculated to be 7:1. Moving forward, this method can be compared with other gear ratios to determine the best fit for the steering system.

### Throttle Sub-Assembly – Elise Chantegros and Natasa Peric

Calculation - Elise Chantegros:

The goal of this mathematical model is to determine the range of the pedal force needed using the NEMA 23 stepper motor to reach full throttle.

Variables:

- Fpedal= 19.06 N

- rspool=0.01 m

- dpedal=0.0635 m

- dcable=0.039 m

- τstepper=1.2Nm [86]

- Fcable=2 kg=9 N

Equations:

- τpedal=Fpedal∙dpedal

- τcable=Fcable∙rspool

- Fscale,max=τstepper, dynamicrmotor∙dcabledpedal [87]

Results and Conclusion:

The stepper motor can sustain between 20-30N of force on the pedal to reach full throttle.

Calculation – Natasa Peric:

This mathematical analysis answers the question of how fast the velocity of the throttle cable is based on its displacement and the stepper motor’s response time. This calculation is crucial in determining what value the range of the velocity must be in order to accurately overcome the stepper motor’s torque.

Variables:

- Θ=22.7°

- rspool=0.02m

- Ispool=15.1E-5kg∙m2

- Istepper=5.3E-5kg∙m2

- dcable=0.0314 m

- Tstepper=1.25 N∙m

- Tspring=0.03924 N∙m

- Tfriction=0.05 N∙m 								   [88]

Equations:

- s=θ∙dcable									   [89]

- Itotal=Ispool+Istepper

- Tnet=Tstepper-Tspring-Tfriction

- tcable=2∙s*Itotalrspool∙Tnet

- vcable=dcabletcable

Results and Conclusion:

The cable’s velocity will be approximately 0.46 - 0.66m/s for a 23 – 33 ms timeframe. This velocity will help determine the response time range for the electrical input to accurately take feedback from the control system and direct it to the throttle response system. Moving forward, the sub-team must determine the viable RPM that can be controlled by the driver.

# Design Concepts

## Functional Decomposition

The team is dedicated to designing and building a fully functional autonomous go-kart by incorporating an integrated and thorough strategy for the braking, steering, and throttle subsystems. The braking system prioritizes safety and rapid response, fulfilling key customer requirements such as a dependable emergency stop mechanism, a response time of 500 milliseconds, and consistent performance. The steering subsystem focuses on achieving precise navigation, potentially incorporating advanced features like robotic feedback for improved safety, while the throttle subsystem ensures controlled acceleration and seamless integration with both autonomous and driver-assisted modes. A functional decomposition chart delineates these subsystems into core functions sensing, decision-making, actuation, and monitoring, providing a structured framework for the design process. This comprehensive methodology, reinforced by the House of Quality, bridges customer expectations with technical execution, positioning the go-kart for successful and reliable closed-course demonstrations as stipulated.

Figure 5: Functional Decomposition

### Brakes Sub-System

Within the functional decomposition for the Autonomous Braking System, the top-level system is divided into three core subsystems: Motor, Power Source, and Braking Mechanism. The Motor subsystem incorporates a 12V or 24V DC lithium battery managed by an Arduino Nano, ultimately controlling the motor, while the Power Source subsystem supplies stable 12 V or 24 V power capable of supporting both continuous and peak demands. The braking mechanism centers on a mechanical caliper directly actuated by a high-torque servo motor, converting electronic commands into physical clamping force on the disc. This structured decomposition ensures clear functional separation that enhances the system's reliability and safety, allowing seamless integration with the broader autonomous navigation architecture.

Figure 6: Brakes Functional Decomposition

### Steering Sub-System

The functional decomposition chart outlines Steering System Operation with sub-functions tailored to autonomous navigation. Sensing Environment includes detecting path deviations and obstacles, using sensor data to maintain accurate tracking. Decision-Making and Control processes this input to adjust steering angles, potentially incorporating robotic feedback mechanisms for enhanced driver alerts. Actuation of Steering engages actuators to execute turns, ensuring smooth lane-keeping, while Monitoring and Feedback provide real-time adjustments and failure detection. This structure is vital as it ensures precise navigation, aligns with the project’s autonomous goals, and supports integration with other subsystems, minimizing errors during complex maneuvers.

Figure 7: Steering Functional Decomposition

### Throttle Sub-System

The throttle subsystem’s functional decomposition focuses on throttle system operation which is divided into three important sub-functions. Sensors include environment sensing by which the system observes speed and terrain to guide throttle adjustment for controlled acceleration. The control system incorporates a Raspberry-Pi microcontroller to transmit information between each electrical component. The mechanism itself largely relies on correct decision-making and controls for autonomous or driver-assisted modes, optimizing power delivery. The actuation itself activates the throttle mechanism to ensure a quick response, while the monitoring and feedback oversee performance and issues. This breakdown is vital as it facilitates smooth mode changes, integrates with braking and steering systems, and guarantees the go-kart meets performance goals during demonstrations, thereby boosting overall reliability.

Figure 8: Throttle Functional Decomposition

## Concept Generation

To develop an effective system, the concept generation process is divided among the three sub-teams, brakes, steering, and throttle. Using the established customer and engineering requirements as a foundation, each sub-team conducted research, gathered input from mentors and clients, and explored a range of possible design solutions for the sub-team. The goal was to compare and evaluate concepts based on performance, cost, safety, and feasibility before picking the best options. The following sections outline the concept generation process and evaluations done by each sub-team, highlighting the advantages and disadvantages of each design.

### Brakes

The brake subsystem design process began with a morphological matrix that identified all required functions for Prototype 1, leading to the development of four distinct concepts. Design 1 employed a linear actuator to directly press the brake pedal, mimicking human operation; although simple, it obstructed the pedal during autonomous mode and prevented simultaneous manual control. To resolve this interference, Design 2 introduced a parallel hydraulic circuit with a secondary master cylinder, allowing autonomous braking while preserving full driver access to the original pedal, but at the cost of added weight, complexity, and potential leak points. Design 3 refined the parallel-circuit approach by replacing the secondary master cylinder with a fast-acting solenoid valve for improved response time and repeatability; however, it still required irreversible modifications to the stock hydraulic system and introduced safety concerns related to fluid integrity.

After evaluating response time, packaging constraints, reliability, ease of implementation, and compliance with client requirements for seamless manual and autonomous use, the team selected Design 4 as the final choice. This design utilizes an Arduino Nano-controlled 350 kg*cm high-torque digital servo that pulls a steel cable wound on a spool attached to the mechanical brake lever of the caliper. By bypassing the hydraulic system entirely and actuating the caliper mechanically, Design 4 eliminates interference with driver input and simplifies installation and maintenance. This provides precise positional control and ensures maximum safety and reliability for both autonomous and manual operation.

Figure 9: Brake Actuator Design 1

Figure 10: Brake Booster, Design 2

A			    	    B				          C

Figure 11: Precision Brake Booster, Design 3

A			           B			    C			    D

Figure 12: Secondary Caliper, Design 4

A 				       B 				        C

Table 1: Brakes Morphological Matrix

### Steering

Generating designs for the steering system, the sub-team gathered ideas based on client and mentor suggestions into a morphological matrix, as shown in Figure 15. Looking into purchasing a power steering conversion kit, the team considered both electronic and hydraulic power kits shown below in Figure 12 and Figure 13. After determining the various costs, the steering sub-team researched a DIY electric power steering (EPS) system that could be customized and was more affordable. The advantage of the EPS is that it has fewer components involved, but it is more expensive than the other options. The hydraulic power steering (HPS) allows precise control; however, the system is flammable, while the others are not. Finally, the DIY EPS system is customizable and cost-effective. However, this system will test our knowledge and might be more time-consuming to complete.

Figure 13: Option 1 - Electrical Power Steering (EPS) Kit

Figure 14: Option 2 - Hydraulic Power Steering (HPS) Kit

Figure 1515: Option 3 - DIY EPS Kit (outdated sketch)

Table 2: Steering Morphological Matrix

### Throttle

For the throttle control system, the sub team generated concepts based on the customer and engineering requirements and created a morphological matrix to decide on three designs. The three main options are a rotary servo with a potentiometer (Figure 15), a linear actuator (Figure 16), and a stepper motor with a potentiometer (Figure 187. The rotary servo offered high accuracy, efficiency, and rapid acceleration, but it was the most expensive option and required a more complex controller. The stepper motor was a low-cost and reliable alternative based on its high torque and accuracy abilities. While the linear actuator has a built-in potentiometer and a simpler design, the stepper motor ultimately offered a more straightforward solution for the team to retrofit an actuation system onto the existing throttle mechanism. Because the motor has a shaft, 3D printed components will be more easily added to the overall design compared to the enclosed structure of the linear actuator, which would make it easier for the team members to alter based on new iterations of the overall design.

Figure 16: Rotary Actuator

Figure 17: Linear Actuator

Figure 18: Stepper Motor

Table 3: Throttle Morphological Matrix

## Selection Criteria

For concept selection, each sub-team developed selection criteria based on the customer and engineering requirements to ensure that all the proposed concepts met project expectations. These criteria were chosen to be measurable and directly related to the system’s performance and integration. Factors such as weight, cost, complexity of integration, efficiency, safety, and user interaction were analyzed using specifications and design considerations. Evaluating these parameters allowed the team to compare the advantages and disadvantages of each concept and determine which design best satisfied the overall project goals and system compatibility.

### Brakes

The brake sub-team evaluated all concepts against six criteria speed, reliability, cost, user interference, ease of installation, and electrical integration with speed, reliability, and zero user interference were ranked as the highest priorities due to their direct impact on safety and client requirements for seamless manual/autonomous switching. A fast response minimizes stopping distance, high reliability (fewest failure points and no fluid dependencies) prevents catastrophic loss of braking, and zero user interference ensures the driver can always override the system instantly. While earlier hydraulic parallel-line designs (secondary master cylinder or solenoid valve) scored well on speed and non-interference, they introduced irreversible modifications, potential leak risks, and higher complexity. After thorough trade-off analysis, Design 4 the cable-driven mechanical lever actuated by an Arduino Nano-controlled 350 kg*cm high-torque servo was selected because it excels in the three primary criteria (fastest practical response, highest reliability with only one mechanical path, and absolutely no interference with manual braking) while remaining cost-effective, easy to install, and simple to integrate electrically, making it the optimal solution for Prototype 1.

### Steering

When choosing between power steering kits, customizability, mass weight, complexity of integration, cost, maneuverability, and user interference were analyzed. Customizability is important because the part will need to be altered for it to be properly integrated into the go-kart. The weight of the system is important because it can affect the movement of the vehicle. The complexity of integration can affect the amount of time and effort that will go into utilizing the system. Cost is considered because staying within budget is a top priority. The maneuverability of the kit ensures that it can respond best to electrical components. User interference determines how well the user will be able to use the system.

### Throttle

When choosing between the listed throttle actuation concepts, specifications on accuracy, efficiency, integration complexity, and cost were heavily considered. Accuracy and integration complexity were the most important to ensure a precise throttle response that can be easily altered. Efficiency impacts overall system performance and power usage, and integration complexity affects the time and effort required for implementation. Cost is a key factor to stay within budget, and self-locking capability ensures the throttle holds position without continuous power. Based on these criteria, the stepper motor was selected as the best choice to begin the actuation design for the throttle mechanism.

## Concept Selection

To identify the best design for each subsystem, the team used Pugh charts and decision matrices. Each sub-team compared its proposed concepts against a defined set of selection criteria derived from the customer and engineering requirements. Weighted factors were applied to reflect the importance of each criterion to ensure that both technical performance and practical implementation were considered. The Pugh charts provided a qualitative comparison between concepts, while the decision matrices quantified the results to support a final decision. The highest-scoring concepts from each sub-team were then modeled in SolidWorks, shown in the following figures.

### Brakes

The criteria used for the brakes concept selection are speed, reliability, cost, user interference, ease of installation, and electrical integration. Speed, reliability, and user interference are of the utmost importance for this project, giving these 3 are critical to the success of the sub-system. Speed will determine how quickly the vehicle can begin braking, which could be the difference between a collision and a complete stop. Reliability is also important since during operation, the team would not be able to safely work on the vehicle, and avoiding system failures will be crucial in determining the success of the project. It also considers the number of failure points that are involved with the design, since the fewer possible means of failure, the better for the vehicle. User interference is one of the biggest challenges to overcome for creating autonomy in the budget provided, and it is one of the clients’ criteria needed for success. It is also apparent that the more control users have over the vehicle, the safer it is in instances of failed autonomy. Cost, ease of installation, and electrical integration are still important factors that also drive the selection process. Ultimately, the design chosen was the booster with a solenoid since it would be the fastest, most reliable, and most user-friendly. This is at the cost of price and ease of installation, so considerations must be made for the time needed to work on prototyping.

Table 4: Brakes Pugh Chart

Table 5: Brakes Decision Matrix

Figure 19: Machinable Brake Line Base Plate

### Steering

In the Pugh Chart and the Decision Matrix, complexity of integration and maneuverability were rated 10% in the decision matrix because it would be easier to work around if the selected design were to score low. The rest were equally rated at 20% because they are harder to work around and all are just as important. The electric power steering kit was chosen as the datum in the Pugh chart because it would be the ideal system to work with. However, the DIY kit scored higher in comparison because of its lower cost and customizability. The scores for these criteria were why the DIY kit scored the highest in the decision matrix as well. A 3D CAD model of the design for the DIY EPS kit can be seen in Figures 19 and 20. The electromatic clutch was created by user Kutuzov1986 [90], and the servo motor was created by user Gharahi [91] from the website GrabCAD.

Table 6: Steering Pugh Chart

Table 7: Steering Decision Matrix

Figure 20: Isometric View of Current Assembly for Steering Prototype

Figure 21: SolidWorks Drawing with BOM

### Throttle

The following tables depict the Pugh Chart and Decision Matrix designated for the throttle subsystem that was used to determine the proper item selection based on the sub-team’s designated criteria. The lowest weighted items for throttle in the Pugh Chart and Decision Matrix were efficiency and power consumption at 10% due to the throttle’s nature of requiring a substantial amount of power to allow the go-kart to move. The highest weighted criteria were reliability and response time at 25% because safety was considered the most significant factor when determining the throttle mechanism. In comparing the different actuation options, the stepper motor paired with a potentiometer displayed the most promising results based on the highest weighted criteria. It is most cost-effective and offers a stable response time while providing enough power for the throttle actuation system to work accurately and safely.

Table 8: Throttle Pugh Chart

Table 9: Throttle Decision Matrix

Figure 22: Stepper Motor Mechanism

Figure 23: Technical Drawing of Stepper Motor Mechanism

# Schedule and Budget

This section outlines the important logistical components that will aid in the support of the success of the capstone project, including scheduling, budgeting, and material preparation. First, a detailed Gantt chart for this current semester and a draft for the upcoming semester, along with a work-breakdown-schedule and a description of each task, will be discussed. Next, the overall project budget will be summarized, going in depth about covering prototyping costs, electrical costs, travel, and any anticipated miscellaneous costs. Finally, a comprehensive Bill of Materials (BoM) will discuss each sub-team’s current item specifications, unit costs, and total costs.

## Schedule

The team’s schedules for the fall and spring semesters are shown below in figures 23 and 24, respectively, within Gantt charts. The main assignments for the fall semester include presentations, reports, and prototypes. These assignments focused on progress made throughout the semester and allowed for answers to questions within the sub-teams. Each round of these assessments was followed by a peer evaluation to determine whether each group member was contributing evenly or not. Within the sub-teams, there were specific topics that required research, with each team having an initial and final design with iteration in between.

Figure 24: Overview of Fall Semester's Schedule

The spring semester consists of the final assignments demonstrating the completion of the project. This includes the engineering model summary, project poster, testing plan, CAD packet, report, and website check. The general goal is to complete the mechanical elements before the spring semester. The goal of the spring semester is to prepare for the undergraduate symposium, where the team will present to multiple forms and groups of people.

Figure 25: General Outline of Spring Semester's Schedule

## Budget

To date, the team has raised $8,473.16 in total funding, including $3,473.16 from donations and fundraising, and the $5,000 sponsorship from Boeing. Of this amount, we have spent $2,399.28, leaving a remaining balance of $6073.88. Sub-team expenditures include $212.51 for brakes, $173.04 for throttle, and $185.12 for steering. Additionally, we purchased the go-kart for $1,600, and have $228.61 in miscellaneous expenses such as safety equipment, garage supplies, graphic design services, and fuel. These values represent our current financial status but will continually change as additional fundraising and donations are received, and more items are purchased.

Our estimated total project budget is $7,900, which includes $2,800 for electrical components, $800 for brakes, $1,000 for steering, $600 for throttle, $400 for travel, $1,800 for the go-kart, and $500 for miscellaneous costs. With total funding currently exceeding our projected expenses, we are financially on track and have a sufficient buffer for unexpected costs.

Although we are in a strong financial position, the team will continue fundraising through our GoFundMe and percentage-night events. This allows the team to remain adaptable as the budget changes with new donations, design iterations, or unexpected costs, ensuring we have the resources needed to complete the project successfully.

## Bill of Materials (BoM)

### Brakes

Table 1: Brakes Bill of Materials below reflects prototype 1’s configuration, relying on a cable-driven mechanical disc brake, actuated by a single high-torque RC servo motor. The complete list includes the S350 RC servo, Grehua mechanical caliper, an Arduino Nano for control, stainless steel brake cable, custom base plate, motor housing, spool, electrical wiring, and all required bolts, nuts, washers, and crimp tube.

Table 10: Brakes Bill of Materials

### Steering

For the current steering prototype plan, the team decided to assemble their BoM with a rotary encoder, mounts, mounting hardware, electrical box, Arduino nano, wiring harness, geared DC motor, a potentiometer, and a timing belt. The quantities of each item and their associated estimated price can be seen in Table 11: Steering Bill of Materials below. According to the BoM, the steering sub-team is staying below its assigned budget.

Table 11: Steering Bill of Materials

### Throttle

The Bill of Materials (BoM) for the throttle incorporates the existing purchases made for the throttle design, as well as any potential components that may be changed for future iterations. The total has decreased from the first listed set of materials because of the change in actuation mechanism from a linear actuator to a stepper motor. The rest of the list includes the 3D printed spool, remaining components, and mounting materials.

Table 12: Throttle Bill of Materials

| Throttle System Requirements |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BoM Level | Part # | Description | Qty | Units | Unit Cost | Total Cost |
| 1 |  | Actuation System |  |  |  |  |
|  | NEMA2 | Stepper Motor | 1 | 1 | $32.88 | $32.88 |
| 2 | SP1 | Stepper Winding Spool | 1 | 1 | $5.99 | $5.99 |
| 2 | TL-LK | Throttle Linkage ( i.e., Clevis and Bracket) | 2-3 | 2-3 | $8.99 | $26.97 |
| 2 | 10KP | 10K Rotary Potentiometer | 1 | 2 | $1.99 | $3.98 |
| 3 |  | Electronics |  |  |  |  |
| 3 | DM0 | DM556T Driver | 1 | 1 | $19.65 | $19.65 |
| 3 | NANO | Nano Microcontroller | 1-2 | 2 | $9.99 | $19.98 |
| 3 | 12VPS | Power Supply | 1 | 1 | $15.99 | $15.99 |
| 4 | EW0 | Electrical Wire | 7-8 | 1 | $7.99 | $7.99 |
| 4 | FUS-7A | Safety Fuse | 1 | 1 | $6.95 | $6.95 |
| 4 | MH0 | Mounting Hardware | 8-12 | 1 | $3.99 | $19.99 |
| Total Number of Parts |  |  | 24 | Total Costs |  | $160.37 |

# Design Validation and Initial Prototyping

To present the team’s ongoing validation and analysis efforts, this section will begin with a detailed Failure Modes and Effects Analysis (FMEA) that identifies critical potential failures, associated risks, and design strategies to mitigate them. This FMEA section will be split by sub-team and will focus on the items chosen for each sub-function’s prototypes. Afterwards, initial prototyping will be discussed by breaking the section up into sub-teams. This portion of the section aims to clarify what question was trying to be answered, what the answer was, and how it informed the design. A summary of all engineering calculations performed since the concept selection phase is also provided to document any progress that has taken place mathematically. Finally, the section concludes with an overview of some potential testing procedures that could be completed in the future, broken up by sub-team.

## Failure Modes and Effects Analysis (FMEA)

### Brakes

Brakes used their BOM to create an FMEA analysis to predict part failures and minimize the wasted time and cost associated with failure. Due to the simplicity of the design used for autonomous braking, there are only a few parts, most of which with catastrophic failures. This danger is somewhat mitigated by the fact that there should be routine inspections before operation, and almost all failures can be avoided. In addition to this, those parts that are susceptible to sudden failure can be aided by simply using the manual brake once the failure is recognized.

Figure 26: Brakes FMEA

### Steering

The steering FMEA analyzed each part mentioned in the subfunction’s BoM for the sub-team’s current prototype and used the FMEA to determine potential failures, potential effect(s) of failure, its severity based off these failures, potential causes and mechanisms of failure, a rating of occurrence for these causes, the current design controls test, a rating of detection based off of the current design controls test, the risk priority number calculated from the ratings of the severity, occurrence, and detection, and lastly a recommended action. The FMEA has prepared the steering sub-team by providing an opportunity to analyze important potential failures and issues that could arise during prototyping.

Figure 27: Steering FMEA

### Throttle

Our team performed an FMEA to identify the most critical risks in the autonomous throttle sub-system and determine how to mitigate them. The highest priority issues were associated with the stepper spool (RPN 324) and rotary potentiometer (RPN 320), both of which could cause loss of throttle control if the cable jammed or the pedal angle was read incorrectly. To address this, we can select a thicker, vibration-resistant cable, add retention features to prevent the spool from slipping, and design to custom rigid mount to secure the potentiometer.

Other components, such as the stepper motor, throttle linkage, and electrical wiring, had moderate RPN values due to risks like overheating, vibration-induced loosening, and wiring failures. These can be mitigated through the addition of a protective fuse, a more stable 3D printed linkage mount, and automotive-grade wire routing away from heat and moving parts.

The team balanced safety, cost, manufacturability, and reliability when determining mitigation strategies. Some solutions, such as custom mounts and upgrading wiring, would increase material costs and assembly time but significantly reduce the likelihood of unsafe throttle behavior. By prioritizing components with the highest RPN values, the team ensures that the subsystem remains reliable.

Figure 28: Throttle FMEA

## Initial Prototyping

### Brakes

For the braking sub-team’s first prototype, the question that needed an answer was “How and Where can the autonomous brake be mounted?” To answer this question, the brake caliper was affixed to the underside of the vehicle, located where the mudguard sits, with a series of four L brackets that could be permanently fixed to the kart. As for the base plate, it was mounted to the frame near the rear left wheel, where it has enough space for the motor and spool to be tied to the caliper lever. Once this prototype was complete, the next step was figuring out what question needed to be answered with prototype 2. The next step for the brake sub team was, “Can the brakes be controlled remotely?” To answer this question, the mounted setup used in prototype 1 was reused. From there, the motor will be attached to electronics that can be controlled remotely.

### Steering

The steering sub team’s goal for prototype 2 was to answer the following question: How to mount the various parts of the prototype to the steering subsystems and the go-kart? The team aimed to answer this question using 3-D printed models of the electrical box and motor casings. Issues that came about during prototyping include tolerance errors, warping, and the angle needed to mount the motor casing to the frame of the kart. From this round of prototyping, the steering team was able to answer the type of clamps to use for mounting, increasing the ventilation for motor casing, mounting for gear system, and other main components, finally allowing the team to move forward with material selection for future prototypes. These results will allow the team to move forward confidently with our mounting dimensions, 3-D printing processes, force analyses, and remodeling within CAD. Moving forward, the team will be triple-checking calculations and measurements to ensure all future sizing will be correct before manufacturing. Along with our tolerances being corrected, extra material will be added to 3-D printed models to prevent wrapping within our designs. As the team develops prototype 2, FMEA analyses will need to be used to ensure that we choose a material for prototype 2 that stands up to the applied forces within the system. Completing justification calculations for our future design will allow the team to continue improving the overall design.

### Throttle

The prototype was created to determine how to mount the stepper motor to the go-kart in a way that doesn’t interfere with existing components. The team needed to identify the mounting location and hardware configuration that would allow the stepper, spool, cable, and encoder to function reliably without obstructing the pull-start mechanism or engine components.

The stepper was initially placed on the back bar parallel to the throttle lever, but this location lacked stability and would likely fall due to vibrations when the kart is running. Then, the stepper was turned upside down from the overhanging rack, parallel to the throttle lever on the opposite side. The wire connects the throttle lever to the spool on the stepper. To stabilize this design, a corner bracket was added to allow for the spool and encoder to hang below.  Additionally, two L-brackets were bolted together, and the corner bracket was attached to the other end to allow for a stable base. This mount was then fully bolted to a steel sheet attached to the top rack with Styrofoam underneath to reduce vibrations. The encoder was connected to the spool and held in place using a thin aluminum sheet to ensure that the encoder doesn’t change direction. This configuration provided significant clearance from the pull-start while maintaining stability.

The prototype confirmed that the location is a major factor in the mounting design. In the future, the prototype will be tested during transportation to ensure that the mount is stable enough to withstand vibration forces. Moreover, the team plans to refine the bracket geometry and improve the reinforcement of the steel mounting plate. Future iterations will also evaluate cable tension, bracket damping, and vibration resistance to finalize a robust mounting system for autonomous throttle control.

## Other Engineering Calculations

### Brakes

Jacob Keeland

The calculations were performed in the service of systematically determining the minimum actuator performance required to achieve full braking or rear-wheel lock under worst-case braking conditions for a fully loaded 233 kg go-kart at 9.5 km/h. Starting from the dynamic rear axle load transfer and progressing through the entire brake actuation chain. These step-by-step derivations quantified the maximum available rear braking force, necessary rotor torque, pad clamping force, cable tension, and finally the servo spool torque of only 5.93 Nm needed to lock the rear wheels. By comparing these requirements against the servo, rated at 35 Nm peak torque and scaled to 72 ms actuation time, factors of safety of 5.90 on torque and 2.08 on time were established, proving a substantial performance margin even under adverse conditions such as pad wear or voltage drop. Ultimately, the calculations analytically validated that the selected single high-torque servo with a mechanical caliper, with significant headroom for future higher-speed operation, confirms that no design changes are required.

Variables:

| Parameter | Symbol | Value | Rational |
| --- | --- | --- | --- |
| Wheelbase: | Lw | 1259.5 mm | Measured and confirmed by seller |
| CG height above ground: | hcg | 0.45m | Measured from driver seated position |
| Static rear weight distribution: | z | 54% | Measured on level ground with driver |
| Coefficient of Friction (tire-road): | μt-r | 0.90 | Dry asphalt |
| Coefficient of Friction (pad-rotor): | μp-r | 0.42 | Semi-Metallic pad avg. |
| Effective rotor radius: | r | 62mm | Measured on installed Mechanical caliper |
| Mechanical Caliper Lever Arm: | La | 50mm | Measured on installed Mechanical caliper |
| Pad Push Distance from Pivot: | Lp | 22mm | Measured on installed Mechanical caliper |
| Caliper Lever’s Mechanical Advantage: | Ma | 50/22= 2.27 | Direct from lever measurement |
| Cable System Efficiency: | η | 0.94 | Estimate |
| Spool Radius: | rs | 13.75mm | Calculated |
| Servo Supply Voltage: | V | 24v ; 50Ah | Lithium |
| Servo Peak Torque: | τmax | 35Nm | Docyke Data Sheet |
| Servo no load speed (18v): | vno load | 0.16 s60° | Docyke Data Sheet |

Equations:

τrotor=Fmax× r

Fmax=μt-r× Fz

Ftotal-clamp= τrotor2μp-r*r

ΔFz=mahcgLw

Factorsafety (time)= tactuation τreq

τreq=T*rsη

Factorsafety= τmaxτreq

T=Ftotal-clamp2Ma

Results:

A single Docyke 350 kg*cm servo pulls a cable to clamp a mechanical caliper, locking the rear wheel of our 233kg go-kart. Math proves the motor only needs 5.9Nm and 72ms, yet the servo delivers 35 Nm in under 80ms, giving us massive safety margins and zero need for design changes. A cheaper motor is in the back of our minds as an area for future optimization.

### Steering

Desirae Mangum

The goal of this calculation model was to justify the design of a 1:1 ratio for a helical gear system that will be used for the steering actuator. This gear ratio will allow the team to have a directly driven system from the motor to the steering rod.

Variables:

- Normal Diametral Pitch, Pn=430.53mm

- Number of Teeth, N=40teeth

- Pitch Diameter, dP=60mm

- Helix Angle, α=38.12°

- Center to Center Distance, C=76.2mm

- Verified Pitch Diameter, D=60mm

- Outside Diameter, O = 93.14mm

- Verified Number of Teeth, NT=40teeth

- Lead of Helix, L= 305.05mm

Equations:

Pn=NdP  						[53]

cosα=2N2PnC

D= NPncosα

O=D+2Pn

NT=Ncos3α

L=πDcot(α)

Results:

With the numbers found through observation and other calculations, the team was able to justify the 1:1 ratio for a set of right-hand driven helical gears with 40 teeth. Moving forward, this justification will allow the steering team to move forward with gear designs for prototype 2.

### Throttle

Natasa Peric

The following calculation depicts the RPM that the stepper motor will undergo using the dimensions and design of the spool that is directly attached to the stepper motor. This information will be crucial for the accurate control over the motor using the chosen driver.

Variables:

| Measurements: | Values | Units |
| --- | --- | --- |
| Fpedal, min = force read from fish scale (2kg) | 19.6 | N |
| dpedal = distance the pedal travels from 0-100% throttle (2.5in) | 0.0635 | m |
| dcable = distance the cable travels when the pedal is pressed from 0% to 100% throttle | 0.0314 | m |
| Tmotor, max = Holding torque of stepper [92] | 2.4 | Nm |
| conservative value of dynamic torque [93] | 0.5 |  |
| dynamic torque | 1.2 | Nm |
| Max turn of spool | 90 | degrees |
| Angle of throttle lever turn | 22.7 | degrees |

Equations:

rspool=dcableθ	          τpedal=Fpedaldpedal	         Fcable=Tpedaldpedal	       τcable=Fpedal∙rspool  [94]

s=θ∙rspool		Itotal=Ispring+Imotor	τnet=τstepper-τspool-τfriction     [95]

t=2∙s∙Itotalrspool∙τnet		νcable=dcabletresponse		RPM=νcable∙60π∙dspool   		         [96]

Mathematical Computation:

F_pedal = 19.6; 	% Units: Newtons (N)

d_pedal = 0.0635; 	% Units: Meters (m)

d_cable = 0.0314; 	% Units: Meters (m)

Tmax_motor = 2.4; 	% Units: Newton-meters (Nm)

v_dynamicT = 0.5; 	% Conservative value of dynamic torque

Tdynamic_motor = 1.2; 	% Dynamic/Useable Torque of Motor

degree_lever = 22.7; 	% Angle of throttle lever turn in degrees

degree_spool = 90;	% Angle the spool will turn in degrees

I_motor = 0.000053;	% Inertia of Stepper Motor in kg*m^2

T_stepper = 1.2;	% Useable Torque in Stepper in Newton-meters (Nm)

T_spring = 0.03924;	% Torque of Return Spring in Newton-meters (Nm)

T_friction = 0.05;	% Estimated Torque due to ground in Newton-meters (Nm)

rad_lever = deg2rad(degree_lever)

rad_spool = deg2rad(degree_spool)

r_spool = d_cable./rad_spool

d_spool = r_spool*2

I_spool = 1.5*(r_spool^2)	% Inertia of Spool in kg*m^2

T_pedal = F_pedal*d_pedal

F_cable = T_pedal./d_pedal

T_cable = F_pedal*r_spool;

T_spool = T_cable

s = rad_lever*r_spool

I_total = I_spool + I_motor

T_net = T_stepper - T_spring - T_spool

t = sqrt((2*s*I_total)./(r_spool*T_net))

v_cable = d_cable./t

RPM = (v_cable*60)./(pi*d_spool)

Results and Conclusion:

For this analysis, the RPM of the spool using a 20 mm was 578rpm with a 0.43 Nm torque. This value was lower than the torque needed for a spool with a 25 mm radius from the initial assumption. Using this value of RPM, the team can determine the correct code needed for the driver in order to ensure that the stepper can achieve that 90-degree turn accurately. Because this torque allows for a high response time in the motor, the driver and encoder must communicate correctly so that the right turn angle is applied to the stepper based on the corresponding command.

## Future Testing Potential

For testing all subsystems, safety is a large concern. No matter what mechanism is tested, there will be a driver wearing a helmet in the vehicle at all times and at least one other team member present. The vehicle will also operate in a closed course with fire extinguishers always accessible should an emergency arise. Currently, the plan is to develop a remote-control system to test the subsystems while programming is still in development. The remote controls will be developed by the team's electrical engineers.

### Brakes

In the future, there will be rigorous testing done to ensure the brake subsystem does not fail and that precautions are put in place such that if failure occurs, it is in a controlled environment. The first article of testing will likely be of the spool. Since the spool is made of PLA, it is perfect for quick testing, but durability will be a factor to consider. It is also a necessary component for the success of the brake caliper; thus, stress testing will be important. Along with the spool there are plenty of factors to be tested such as the braking speed, time needed to brake, and forces within the caliper. To cover all bases of the kart testing will also be done in various conditions such as night, rain, and potentially snow.

### Steering

The purpose of testing the steering system will be to ensure all general and sub-team specific ERs are met. To test these, the right turn and general navigation mechanisms will be observed. Performing a right turn enables the radius, time, and velocity of the turn to be measured and compared to their respective ERs. These can simply be measured with a measuring tape, a stopwatch, and a speedometer. Of course, the right turn will be performed multiple times and at multiple different speeds to ensure the system is fully optimized. To ensure the system can properly navigate the vehicle, it must be able to change directions within the required response time. Again, this will be measured multiple times by a stopwatch and at different speeds. The purpose of recording these data is to ensure the system is safely operational and can quickly navigate a user away from obstacles when needed.

### Throttle

For future testing, our team plans to drive the go-kart under normal operating conditions to evaluate the stability of the stepper motor mount. This will allow us to observe how the mounting hardware responds to vibration, lateral forces, and repeated loading during real-world operation. By collecting data on any displacement, loosening, or structural flexing, we can verify the robustness of our design and identify any necessary improvements before final integration into the autonomous system.

# CONCLUSION

The Boeing Autonomous capstone is a team-based project that focuses on designing an actuation system that can be retrofitted onto a vehicle, in which the team will use a go-kart. Mechanical and electrical components will be incorporated to control the go-kart in an autonomous fashion using a variety of motors and surrounding sensors. These components will work for three main sections of the go-kart, which are the brakes, steering, and throttle. To meet the customer requirements, the team must design the actuation system in such a way that the go-kart can navigate through a predefined route by detecting and avoiding obstacles within 500 milliseconds. Moreover, it must reach a maximum speed of 15 km/hr and run continuously on a single battery change for at least 20 minutes. In this design, fail-safe mechanisms are crucial to ensure that the autonomous and manual modes will keep the passenger safe at all times.

The customer requirements for this project have remained consistent with clients from Boeing, but the engineering requirements vary for each sub-team. Because each sub-team has a separate goal, the engineering requirements differ to ensure that each portion of the go-kart can autonomously work in unison with the other components. Based on iterations made by each sub-team, the House of Quality and Concept Selection have changed to better fit the actuation system for the go-kart. Specifically, the throttle sub-team has changed course on its selected actuation motor based on new qualifications that best fit the use of a stepper motor over the initial proposal of using a linear actuator. These changes have proven to be significant for the sub-team as they have iterated and designed around the stepper motor for a simpler approach that allows them to modify the design more than they could if they used a linear actuator. Hence, such changes have allowed the sub-teams to recognize what mechanisms may seem initially suitable but may not be as effective in results based on redefined calculations and sizing on the go-kart.

The solutions currently proposed for the autonomous go-kart system are divided into three separate methods. For the brakes sub-team, the group plans to mount the motor and mechanical caliper. Then, they will test the actuation mechanism by connecting the pull wire from the spool to the caliper arm. They also plan to use a waterproof enclosure on the motor to protect the assembly. For the steering sub-team, the group plans to design an assembly of a motor, two helical gears, and the electrical components inside a secure box. The motor will control one gear by being directly attached, while the other gear is attached to the steering column to turn the steering wheel. For the throttle sub-team, the group plans to wire a driver and microcontroller to the mounted stepper motor and encoder so that all necessary functions will be relayed accurately and efficiently to the electrical components. To control the front acceleration pedal, an electrical wire will run from the driver and microcontroller to the front end of the go-kart. This wire will connect to the encoder securely attached to the front pedal, which will allow for accurate readings of the pedal angle.

The current outcome for this project includes a secured go-kart with safety features stored on campus, three separate prototypes for the first iteration of each brake, steering, and throttle components, over $3000 raised in donations through GoFundMe, and over $100 earned in restaurant-sponsored events. Challenges that all sub-teams are facing include delays in part deliveries and struggles with the current braking system. The electrical sub-team has collaborated with the corresponding mechanical sub-team to address downfalls that may arise once wiring and electrical connections are integrated further into the actuation system. For Spring 2026, the sub-teams will continue to improve their designs using testing, refining calculations, and designing more accurate CAD models of their components. The electrical sub-team plans to integrate a computer-based monitoring system that will be directly connected to the go-kart while it is being driven. Additionally, a remote-control mechanism will be used before making the go-kart fully autonomous to ensure that there is a strong connection between each of the electrical components so that they can fully control the mechanical parts of the go-kart. For testing, the go-kart will be tested on a closed course using cones to evaluate performance and detection to maximize its safety and reliability.

# REFERENCES

The following information provides a comprehensive list of references, figures, tables, and previous regarding all content associated with this report.

[1] TheRacingXpert, “ What Is The Role Of Steering Angle Sensor In Autonomous Driving?,”YouTube,https://youtu.be/bUD_fHQGjMk?si=sNnU_rH6ueBvyz2l (accessed Sep. 14, 2025).

[2] N. H. T. S. A. part of the U.S. Department of Transportation, “Automated vehicles for safety,” NHTSA, https://www.nhtsa.gov/vehicle-safety/automated-vehicles-safety (accessed Sep. 17, 2025).

[3] J. E. Shigley, C. R. Mischke, and T. H. Brown, Shigley’s Mechanical Engineering Design, 11th ed. New York, NY, USA: McGraw-Hill Education, 2020.

[4] H. Luo, X. Hu, and L. Huang, “A Hybrid Model for Vehicle Acceleration Prediction,” Sensors (Basel, Switzerland), vol. 23, no. 16, p. 7253, 2023

[5] A. Altby and D. Majdandzic, “Design and implementation of a fault-tolerant drive-by-wire system.” Accessed: Nov. 26, 2025.

[6] H. Mauser, E. Thurner, and E. Hw, “Electronic Throttle Control -A Dependability Case Study.” Accessed: Nov. 26, 2025.

[7] H. Guo, D. Cao, H. Chen, Z. Sun, and Y. Hu, “Model predictive path following control for autonomous cars considering a measurable disturbance: Implementation, testing, and verification,” Mechanical Systems and Signal Processing, vol. 118, pp. 41–60, Mar. 2019

‌[8] S. Xu, H. Peng, Z. Song, K. Chen, and Y. Tang, “Accurate and Smooth Speed Control for an Autonomous Vehicle,” IEEE Xplore, Jun. 01, 2018.

[9] N. Kolekar and S. Shelar, “Certified Organization) Website: www,” International Journal of Innovative Research in Science, Engineering and Technology (An ISO, vol. 3297, 2007,

[10] R. Conatser, J. Wagner, S. Ganta, and I. Walker, “Diagnosis of automotive electronic throttle control systems,” Control Engineering Practice, vol. 12, no. 1, pp. 23–30, Jan. 2004,

[11] “Drive-by-wire Conversion for Autonomous Vehicle,” Robotics Knowledgebase, Dec. 07, 2022.

[12] “Digital Twin System for Automotive Safety with Integration of a Safety Controller,” Research portal Eindhoven University of Technology, 2024.

[13] V. S. Dubey, Ruhshad Kasad, and K. Agrawal, “Autonomous Braking and Throttle System: A Deep Reinforcement Learning Approach for Naturalistic Driving,” arXiv (Cornell University), Jan. 2021,

[14] B. Mok, M. Johns, S. Yang, and W. Ju, “Reinventing the Wheel,” Oct. 2017, doi: https://doi.org/10.1145/3126594.3126655.

[15] W. Kim, C. Kang, Y.-S. Son, and C. Chung, “Nonlinear Steering Wheel Angle Control Using Self-Aligning Torque with Torque and Angle Sensors for Electrical Power Steering of Lateral Control System in Autonomous Vehicles,” Sensors, vol. 18, no. 12, p. 4384, Dec. 2018, doi: https://doi.org/10.3390/s18124384.

[16] J. E. Naranjo, C. González, R. García, and T. de Pedro, “Electric Power Steering Automation for Autonomous Driving,” Lecture Notes in Computer Science, pp. 519–524, 2005, doi:https://doi.org/10.1007/11556985_67.

[17] P. Falcone, F. Borrelli, J. Asgari, H. E. Tseng, and D. Hrovat, “Predictive Active Steering Control for Autonomous Vehicle Systems,” IEEE Transactions on Control Systems Technology, vol. 15, no. 3, pp. 566–580, May 2007, doi: https://doi.org/10.1109/tcst.2007.894653.

[18] M. K. Mishra, A. Maurya, and R. R. Dev, “Automatic Steering System,” International Journal of Science and Research (IJSR), vol. 5, no. 5, pp. 105–107, May 2016, doi:10.21275/NOV163265

[19] Z. M. U. Din, W. Razzaq, U. Arif, W. Ahmad, and W. Muhammad, “Real Time Ackerman Steering Angle Control for Self-Driving Car Autonomous Navigation,” 2019 4th International Conference on Emerging Trends in Engineering, Sciences and Technology (ICEEST), pp. 1–4, Dec. 2019, doi: https://doi.org/10.1109/iceest48626.2019.8981710.

[20] “The Motor Carrier Safety Planner,” Dot.gov, 2025. https://csa.fmcsa.dot.gov/safetyplanner/MyFiles/SubSections.aspx?ch=22&sec=64&sub=141 (accessed Sep. 17, 2025).

[21] Z. Mehdiyev, C. Felh&#337;, and K. P. Zolt&aacute;n, “Investigation on 3D printing parameters of PLA polymers for gear applications,” SpringerLink, https://link.springer.com/chapter/10.1007/978-3-031-15211-5_55#citeas (accessed Nov. 24, 2025).

[22] H.-J. Dennig, L. Zumofen, and A. Kirchheim, “Feasibility investigation of gears manufactured by fused filament fabrication,” SpringerLink, https://link.springer.com/chapter/10.1007/978-3-030-54334-1_22 (accessed Nov. 24, 2025).

[23] R. Ciobanu, C. I. Rizescu, D. Rizescu, and B. Gramescu, “Surface durability of 3D-printed polymer gears,” MDPI, https://www.mdpi.com/2076-3417/14/6/2531 (accessed Nov. 24, 2025).

[24] W. F. Milliken and D. L. Milliken, Race Car Vehicle Dynamics. Warrendale, PA, USA: SAE International, 1995.

[25] J. E. Shigley, C. R. Mischke, and T. H. Brown, Shigley’s Mechanical Engineering Design, 11th ed. New York, NY, USA: McGraw-Hill Education, 2020.

[26] R. Limpert, Brake Design and Safety, 3rd ed. Warrendale, PA, USA: SAE International, 2011.

[27] Docyke Technology Company, “350 kg·cm high-torque digital servo datasheet,” 2022. [Online]. Available: https://www.amazon.com/dp/B09XDXSFRR

[28] D. Halliday, R. Resnick, and J. Walker, Fundamentals of Physics, 11th ed. Hoboken, NJ, USA: Wiley, 2018.

[29] DEVMO, “BTS7960B 43A high-power motor driver module,” Amazon product specification, 2023. [Online]. Available: (insert actual Amazon link if you have it, or keep as is)

[30] TrailMaster, “Mid XRX/R Parts Manual,” TrailMaster USA, 2023. [Online]. Available: https://www.trailmasterparts.com/manuals/. [Accessed: Oct. 7, 2025]

[31] Hammerhead Off-Road, “Mudhead 208R Parts Diagram,” Hammerhead Performance, 2022. [Online]. Available: https://hammerheadoffroad.com/pages/parts. [Accessed: Oct. 7, 2025]

[32] HydraForce, “HydraForce 12 Volt DC NC Solenoid Valve SV10-22-0-N-00-9-5112-NC,” Surpluscenter.com. [Online]. Available: https://www.surpluscenter.com/Brands/Hydraforce/Hydraforce-12-Volt-DC-NC-Solenoid-Valve-SV10-22-0-N-00-9-5112-NC.axd. [Accessed: Oct. 7, 2025]

[33] Amazon, “DEVMO BTS7960B 43A Double DC Stepper Motor Driver Module H-Bridge PWM Smart Car Compatible with Arduino, Raspberry Pi,” Amazon.com. [Online]. Available: https://www.amazon.com/DEVMO-BTS7960B-Stepper-H-Bridge-Arduino/dp/B07SZ4T699?adgrpid=177218412183&hvpone=&hvptwo=&hvadid=748029726649&hvpos=&hvnetw=g&hvrand=15004738281654529711&hvqmt=&hvdev=c&hvdvcmdl=&hvlocint=&hvlocphy=1013462&hvtargid=dsa-2418061332904&hydadcr=&mcid=&hvocijid=15004738281654529711--&hvexpln=67&tag=googhydr-20&hvsb=Computers_d&hvcampaign=dsadesk. [Accessed: Oct. 7, 2025].

[34] U.S. Solid, “High Pressure Solenoid Valve - 3/4” 12V DC Brass 50 Bar, 300°F High-Pressure High-Temperature Solenoid Valve,” Ussolid.com. [Online]. Available: https://ussolid.com/products/high-pressure-solenoid-valve-3-4-12v-dc-brass-50-bar-300-high-pressure-high-temperature-solenoid-valve-html. [Accessed: Oct. 7, 2025].

[35] S. G. Shadle, L. H. Emery, and H. K. Brewer, “Vehicle Braking, Stability and Control,” Login - CAS – central authentication service, https://www-jstor-org.libproxy.nau.edu/stable/44668097?seq=6 (accessed Sep. 14, 2025).

[36] K. D. Marshall, R. L. Phelps, M. G. Pottinger, and W. Pelz, “The effect of tire break-in on force and moment properties,” SAE Technical Paper Series, Feb. 1977. doi:10.4271/770870

[37] F. Talati and S. Jalalifar, “Analysis of heat conduction in a disk brake system,” Heat and Mass Transfer, vol. 45, no. 8, pp. 1047–1059, Jan. 2009. doi:10.1007/s00231-009-0476-y

[38] M. Haataja and T. Leinonen, “On the distribution of braking forces in road braking,” SAE Technical Paper Series, Dec. 2000. doi:10.4271/2000-01-3413

[39] P. S. Bokare and A. K. Maurya, “Acceleration-deceleration behaviour of various vehicle types,” Transportation Research Procedia, vol. 25, pp. 4733–4749, 2017. doi:10.1016/j.trpro.2017.05.486

[40] “Autonomous Vehicles Testing and operating in the state of Arizona,” Autonomous Vehicles Testing and Operating in the State of Arizona | Department of Transportation, https://azdot.gov/mvd/services/professional-services/autonomous-vehicles-testing-and-operating-state-arizona (accessed Sep. 17, 2025).

[41] Wikipedia Contributors, “Braking distance,” Wikipedia, Oct. 30, 2019. https://en.wikipedia.org/wiki/Braking_distance

[42] Wikipedia Contributors, “Acceleration,” Wikipedia, Oct. 05, 2025.

[43] Design and analysis of a brake caliper, https://iaeme.com/MasterAdmin/Journal_uploads/IJMET/VOLUME_7_ISSUE_4/IJMET_07_04_024.pdf (accessed Nov. 22, 2025).

[44] C. Saurabh, “Motor Torque Calculations For Electric Vehicle ,” Aug. 2015. https://d1wqtxts1xzle7.cloudfront.net/60814999/Motor_Torque_Calculations_For_Electric_Vehicle20191006-126594-1gwy5v2-libre.pdf?1570384694=&response-content-disposition=inline%3B+filename%3DMotor_Torque_Calculations_For_Electric_V.pdf&Expires=1763840117&Signature=PwJ~bS0J2u8OkmIwRyCcJtZzOKT30HMQkn65ySiqbgCE7uCKcF3NyWY-~2wExuMaUzy9A~W9l8NijbyrmOiGEA3TuPB-rlnVwGoYhqWxWGIbPfJE-aZt~jCHt9HUgRjM5iuG5oZav3nOLTkgZz-FyWGccxyqVhBIBQso080Ji1e4S7c29S~KDVixSKrAsXaPsIbJTvbZN~6GdgrjXJslaX5GagSfHaDEwfRJ18CQcEjVdLP5YuB4niJ1RaMco4txC8hJd7YAxhFIo3y4LmJZHHvjAAgZ-EWlzcZpqK2eIJlMeDrXiqlcQ7WU~JhulolzvmBf7L6KAMFXauERWE65CQ__&Key-Pair-Id=APKAJLOHF5GGSLRBV4ZA (accessed Nov. 22, 2025).

[45] M. J. Lewis, “STEERING SYSTEM AND METHOD FOR AUTONOMUS VEHICLES,” 	Mar. 2022  STEERING SYSTEM AND METHOD FOR AUTONOMUS VEHICLES - EuropeanPatent Office - EP 3283349 B1

[46] “Intelligent Electric Power Steering: Artificial Intelligence Integration Enhances Vehicle 	Safety and Performance,” Intelligent Electric Power Steering: Artificial Intelligence Integration Enhances Vehicle Safety and performance, https://arxiv.org/html/2412.08133v1 (accessed Sep. 14, 2025). Intelligent Electric Power Steering: Artificial Intelligence Integration Enhances Vehicle Safety and Performance

[47] B. Németh, D. Fényes, P. Gáspár and A. Mihály, "Analysis and robust control design of a 	steering system forautonomous vehicles," 2017 IEEE International Conference on Advanced Intelligent Mechatronics (AIM), Munich,Germany, 2017, pp. 535-540, doi: 10.1109/AIM.2017.8014072. keywords: {Vehicle dynamics;Steeringsystems;Frequency-domain analysis;Tires;Autonomous vehicles;Control design}, Analysis and robust control design of a steering system for autonomous vehicles | IEEE Conference Publication | IEEE Xplore

[48] H. Halin et al., "Simulation Studies - Path Tracking of an Autonomous Electric Vehicle (AEV) by Using FuzzyInformation of Speed and Steering Angle," 2018 International Conference on Computational Approach in SmartSystems Design and Applications (ICASSDA), Kuching, Malaysia, 2018, pp. 1-4, doi: 10.1109/ICASSDA.2018.8477593.keywords: {Autonomous vehicles;Navigation;Wheels;Mechatronics;Reliability;Automobiles;Inputvariables;fuzzy;simulation;lateral error;autonomous;steering control}, Simulation Studies - Path Tracking of anAutonomous Electric Vehicle (AEV) by Using Fuzzy Information of Speed and Steering Angle | IEEE ConferencePublication | IEEE Xplore

[49] Y. Jeong, D. H. Kim, M. H. Youn, S. Park, X. J. Li and T. Lee, "Model Predictive Control-Based Path Tracking withFour-Wheel Independent Steering, Driving, and Braking Autonomous Vehicles on Low Friction Road," 2023 23rdInternational Conference on Control, Automation and Systems (ICCAS), Yeosu, Korea, Republic of, 2023, pp. 1427-1432, doi: 10.23919/ICCAS59377.2023.10316779. keywords: {Transmission line matrixmethods;Friction;Roads;Wheels;Predictive models;Tires;Robustness;Autonomous vehicle;Four-wheel independentsteering;Four-wheel independent driving;Four-wheel independent braking;Low friction road. Model predictivecontrol;Control allocation}, Model Predictive Control-Based Path Tracking with Four-Wheel Independent Steering,Driving, and Braking Autonomous Vehicles on Low Friction Road | IEEE Conference Publication | IEEE Xplore

[50] A. Pushpakanth and M. N. Dhavalikar, Development of Steering Control System for Autonomous Vehicle,https://www.ijrte.org/wp-content/uploads/papers/v11i2/B71050711222.pdf (accessed Sep. 15, 2025).  Development of Steering Control System for Autonomous Vehicle

[51] TheRacingXpert, “ What Is The Role Of Steering Angle Sensor In Autonomous Driving?,”YouTube,https://youtu.be/bUD_fHQGjMk?si=sNnU_rH6ueBvyz2l (accessed Sep. 14, 2025).

[52] N. H. T. S. A. part of the U.S. Department of Transportation, “Automated vehicles for safety,” NHTSA, https://www.nhtsa.gov/vehicle-safety/automated-vehicles-safety (accessed Sep. 17, 2025).

[53] 	E. Oberg, Machinery’s handbook, 31st edition, https://www.rexresearch1.com/MathFormulaHandbooksLibrary/MachinerysHandbkOBERG.pdf (accessed Nov. 24, 2025).

[54] 	Designing Made Easy, “Helical Gear Design in SolidWorks: A Step-by-Step Tutorial,” YouTube, https://youtu.be/2u5UtZ1R3_Y?si=u8Lmh5eSjJgTHSdH (accessed Nov. 24, 2025).

[55] R. Nalbandian, T. Blais, and R. Horth, “A Recommended New Approach on Motorization Ratio Calculations of Stepper Motors.” Accessed: Oct. 16, 2025. [Online]. Available: https://ntrs.nasa.gov/api/citations/20150004072/downloads/20150004072.pdf

[56] S. Thapa, “3D Printing of Automobile Power Transmission System Using Tough PLA,” Urn.fi, 2020, doi: https://www.theseus.fi/handle/10024/343865.

[57] M. Venczel, Á. Veress, and Z. Peredy, “Past and Future Practical Solutions for Torsional Vibration Damping in Vehicle Industry,” Periodica Polytechnica Transportation Engineering, vol. 50, no. 4, pp. 318–329, Aug. 2022, doi: https://doi.org/10.3311/PPtr.19194.

[58] Mattikalli, S., Rayar, V., Yallure, B., Akki, S., & Thane, S. (2024). AUTOSAR-enabled throttle position sensor testing and analysis using MATLAB. 2024 IEEE North Karnataka Subsection Flagship International Conference (NKCon), 1–5. https://doi.org/10.1109/nkcon62728.2024.10775293

[59] H. Niaz, Muhammad Abdullah Sheeraz, and M. A. Naeem, “Multi-Parametric Analysis of a Mimicked Accelerating Pedal (Via DC Motor) of an Electric Vehicle,” DOAJ (DOAJ: Directory of Open Access Journals), pp. 48–48, Oct. 2023, doi: https://doi.org/10.3390/asec2023-15253.

[60] Oluwatobiloba Adejumo, “Safety of 3D Printing in Automotive Industries,” SSRN Electronic Journal, Jan. 2025, doi: https://doi.org/10.2139/ssrn.5112375.

[61] J. Lee, K. Oh, and K. Yi, “DEVELOPMENT OF AN EMERGENCY CONTROL ALGORITHM FOR A FAIL-SAFE SYSTEM IN AUTOMATED DRIVING VEHICLES.” Available: https://www-esv.nhtsa.dot.gov/Proceedings/26/26ESV-000101.pdf

[62] Arduino, “Power Consumption on Arduino Boards,” Arduino.cc, 2024. https://docs.arduino.cc/learn/electronics/power-consumption/

[63]“NHTSA Rulemakings Related to Automated Driving System-Equipped Vehicles NHTSA Rulemakings Related to Automated Driving System- Equipped Vehicles,” 2024. Available: https://www.nhtsa.gov/sites/nhtsa.gov/files/2024-05/Report-to-Congress-NHTSA-Rulemakings-Related-to-Automated-Driving-System-Equipped-Vehicles.pdf

[64] Krystian Radlak, M. Szczepankiewicz, T. Jones, and P. Serwa, “Organization of machine learning based product development as per ISO 26262 and ISO/PAS 21448,” arXiv (Cornell University), pp. 110–119, Dec. 2020, doi: https://doi.org/10.1109/prdc50213.2020.00022.

[65] W. Li, Z. Xie and P. He, "Research on the Application of Multi-Sensor Fusion in Autonomous Driving," 2025 5th International Conference on Sensors and Information Technology, Nanjing, China, 2025, pp. 389-392, doi: 10.1109/ICSI64877.2025.11010055.

[66] P. Y. Leong and N. S. Ahmad, "LiDAR-Based Obstacle Avoidance With Autonomous Vehicles: A Comprehensive Review," in IEEE Access, vol. 12, pp. 164248-164261, 2024, doi: 10.1109/ACCESS.2024.3493238.

[67] Y. W. Jeong, C. C. Chung and W. Kim, "Nonlinear Hybrid Impedance Control for Steering Control of Rack-Mounted Electric Power Steering in Autonomous Vehicles," in IEEE Transactions on Intelligent Transportation Systems, vol. 21, no. 7, pp. 2956-2965, July 2020, doi: 10.1109/TITS.2019.2921893.

[68] N. Kalkovaliev and G. Mirceva, "Autonomous Driving by Using Convolutional Neural Network," 2021 3rd International Congress on Human-Computer Interaction, Optimization and Robotic Applications (HORA), Ankara, Turkey, 2021, pp. 1-4, doi: 10.1109/HORA52670.2021.9461350.

[69] D. Meng, H. Chu, M. Tian, B. Gao and H. Chen, "Neural Network-Aided Fast Iterative Model Predictive Control for Autonomous Vehicle Motion Control," in IEEE Sensors Journal, vol. 25, no. 19, pp. 37181-37194, 1 Oct.1, 2025, doi: 10.1109/JSEN.2025.3598267.

[70] L. Anzalone, S. Barra and M. Nappi, "Reinforced Curriculum Learning For Autonomous Driving In Carla," 2021 IEEE International Conference on Image Processing (ICIP), Anchorage, AK, USA, 2021, pp. 3318-3322, doi: 10.1109/ICIP42928.2021.9506673.

[71] S. He, X. Xu, J. Xie, F. Wang and Z. Liu, "Fault Diagnosis Strategy of Autonomous Steering System for Intelligent Vehicle Based on State Estimation," 2022 6th CAA International Conference on Vehicular Control and Intelligence (CVCI), Nanjing, China, 2022, pp. 1-6, doi: 10.1109/CVCI56766.2022.9964723.

[72] D. Kyrychuk and A. Segin, "Training of a Convolutional Neural Network and Its Object Recognition Ability Depending on Illumination and Contrast of Images," 2021 11th International Conference on Advanced Computer Information Technologies (ACIT), Deggendorf, Germany, 2021, pp. 722-725, doi: 10.1109/ACIT52158.2021.9548493. keywords: {Training;Image recognition;Transfer learning;Neural networks;Lighting;Feature extraction;Object recognition;convolutional neural network;image classification;object detection},

[73] L. Bo, W. ShiWei, H. Qinghui, Q. Zan and Y. Jingyi, "An Improved Neural Network Algorithm for Intelligent Object Recognition," 2021 17th International Conference on Computational Intelligence and Security (CIS), Chengdu, China, 2021, pp. 26-29, doi: 10.1109/CIS54983.2021.00014.

[74] S. Tennekoon, N. Wedasingha, A. Welhenge, N. Abhayasinghe and I. Murray Am, "Advancing Object Detection: A Narrative Review of Evolving Techniques and Their Navigation Applications," in IEEE Access, vol. 13, pp. 50534-50555, 2025, doi: 10.1109/ACCESS.2025.3551686.

[75] H. Luo, X. Hu, and L. Huang, “A Hybrid Model for Vehicle Acceleration Prediction,” Sensors (Basel, Switzerland), vol. 23, no. 16, p. 7253, 2023

[76] A. Altby and D. Majdandzic, “Design and implementation of a fault-tolerant drive-by-wire system.” Accessed: Nov. 26, 2025.

[77] “Coleman Powersports 196cc 2-Seater Gas-Powered Go Kart,” Tractor Supply Company, 2025. https://www.tractorsupply.com/tsc/product/coleman-196cc-gas-powered-two-seater-go-kart-gk200 (accessed Nov. 06, 2025).

[78] “Amazon.com: HIAORS 145/70-6 145 70-6 Tire Tubeless for Coleman CC100X CT100U Baja Dirt Bug Doodle Bug DB30 Monster Moto MMB80 MMB105 Motovox MBX10 MBX11 GMB100 RT100 Mini Bike Kazuma Roketa 110cc ATV Quad Go Kart : Automotive,” Amazon.com, 2025. https://www.amazon.com/HIAORS-145x70x6-Tubeless-Motovox-Powersports/dp/B089LY44CF?th=1 (accessed Nov. 06, 2025).

[79] J. Vogel, “Tech Explained: Ackermann Steering Geometry,” Racecar Engineering, Apr. 06, 2021. https://www.racecar-engineering.com/articles/tech-explained-ackermann-steering-geometry/[41] R. Budynas, Shigley’s Mechanical Engineering Design ISE. McGraw Hill, 2024.

[80] “Steering Ratio Calculator - Calculator Academy,” Calculator Academy, Jul. 27, 2023. https://calculator.academy/steering-ratio-calculator/ (accessed Nov. 06, 2025).

[81] University of Guelph, “Torque and Rotational Motion Tutorial | Physics,” www.physics.uoguelph.ca, 2024. https://www.physics.uoguelph.ca/torque-and-rotational-motion-tutorial

[82] Y. Han, L. He, X. Wang, and C. Zong, “Research on Torque Ratio Based on the Steering Wheel Torque Characteristic for Steer-by-Wire System,” Journal of Applied Mathematics, vol. 2014, no. 8, pp. 1–7, Jul. 2014, doi: https://doi.org/10.1155/2014/929164.

[83] “Angular Velocity | dummies,” Dummies.com, 2016. https://www.dummies.com/article/academics-the-arts/science/physics/angular-velocity-148059/

[84] OpenStax, “10.8 Work and Power for Rotational Motion,” pressbooks.online.ucf.edu, Aug. 2016, Available: https://pressbooks.online.ucf.edu/osuniversityphysics/chapter/10-8-work-and-power-for-rotational-motion/

[85] R. Budynas, Shigley’s Mechanical Engineering Design ISE. McGraw Hill, 2024.

[86] R. Nalbandian, T. Blais, and R. Horth, “A Recommended New Approach on Motorization Ratio Calculations of Stepper Motors.” Accessed: Oct. 16, 2025. [Online]. Available: https://ntrs.nasa.gov/api/citations/20150004072/downloads/20150004072.pdf

[87] E. Oberg and C. J. Mccauley, Machinery’s Handbook: a reference book for the mechanical engineer, designer, manufacturing engineer, draftsman, toolmaker, and machinist. New York: Industrial Press, 2012.

[88] H. Olsson, K. J. Åström, C. Canudas de Wit, M. Gäfvert, and P. Lischinsky, “Friction Models and Friction Compensation,” European Journal of Control, vol. 4, no. 3, pp. 176–195, Jan. 1998, doi: https://doi.org/10.1016/s0947-3580(98)70113-x.

[89] J. L. Meriam and L. G. Kraige, Engineering mechanics. Vol. 2, Dynamics. Hoboken, N.J.: Wiley; Chichester, 2012.

[90] “Free CAD Designs, Files & 3D Models | The GrabCAD Community Library,” Grabcad.com, 2025. https://grabcad.com/library/electromagnetic-clutch-2 (accessed Oct. 20, 2025).

[91] “Free CAD Designs, Files & 3D Models | The GrabCAD Community Library,” Grabcad.com, 2025. https://grabcad.com/library/delta-ac-servo-motor-2kw (accessed Oct. 20, 2025).

[92] “Nema 23 Stepper Motor 2.4Nm(339.87oz.in) 4A 57x57x82mm 8mm Shaft 4 Wires - 23HS32-4004S | StepperOnline,” Omc-stepperonline.com, 2023. https://www.omc-stepperonline.com/nema-23-stepper-motor-2-4nm-339-79oz-in-4a-57x57x82mm-8mm-shaft-4-wires-23hs32-4004s

[93] R. Nalbandian, T. Blais, and R. Horth, “A Recommended New Approach on Motorization Ratio Calculations of Stepper Motors.” Available: https://ntrs.nasa.gov/api/citations/20150004072/downloads/20150004072.pdf

[94] H. Olsson, K. J. Åström, C. Canudas de Wit, M. Gäfvert, and P. Lischinsky, “Friction Models and Friction Compensation,” European Journal of Control, vol. 4, no. 3, pp. 176–195, Jan. 1998, doi: https://doi.org/10.1016/s0947-3580(98)70113-x.

[95] E. Oberg and C. J. Mccauley, Machinery’s handbook : a reference book for the mechanical engineer, designer, manufacturing engineer, draftsman, toolmaker, and machinist. New York: Industrial Press, 2012.

[96] J. L. Meriam and L. G. Kraige, Engineering mechanics. Vol. 2, Dynamics. Hoboken, N.J.: Wiley ; Chichester, 2012.

# APPENDICES

## Appendix A: Past Tables, Figures, and References

### Previous Throttle QFD:

### Pugh Chart for Previous Selection Criteria for Throttle Mechanism:

### Decision Matrix for Previous Selection Criteria for Throttle Mechanism:

### Stress and Strength Test for Previous Linear Actuator Selection:

### Technical Drawing for Previous Linear Actuator:

### Top and Right View of Stepper Motor Mechanism

### Isometric View of Spool

### Top and Front View of Spool

### Technical Drawing of Spool

### Steering System QFD

### Steering Prototype 1 CAD Model

### Steering Prototype 1 CAD Model Drawing

## Appendix B: Breakdown of Morphological Matrix

Figure 9a: 24V DC Motor [97]

Figure 9c: On/Off Solenoid (Manual) [98]

Figure 10d: H-Board [99]

Figure 11a: Push Rod [100]

Figure 11d: 24V DC Motor [101]

Figure 11f: On/Off Electric Solenoid [102]

Figure 11g: Raspberry-Pi [103]

Figure 14: Rotary Actuator [104]

Figure 14a: ECU [105]

Figure 14b: Motor Controller [106]

Figure 14c: Servo Controller [107]

Figure 14d: DC Motor [108]

Figure 14e: Hydraulic Pump [109]

Figure 14f: Servo Motor [110]

Figure 14g: Current Sensor [111]

Figure 14h: Torsion bar and Rotary Valve [112]

Figure 14i: Potentiometer [113]

Figure 12 – Option 1: Electric Power Steering (EPS) Kit [114]

Figure 12 – Option 2: Hydraulic Power Steering (HPS) Kit [115]

Figure 15: Linear Actuator [116]

Figure 16: Stepper Motor [117]

Figure 17a: H-Bridge Board [118]

Figure 17c: Arduino Nano [119]

Figure 17g: Potentiometer [120]

Figure 17h: Rotary Encoder [121]

Figure 17i: High-Power Killswitch [122]

Figure 17j: Limit Switch [123]

Figure 17k: Low-Power Killswitch [124]

## Appendix C: References for Morphological Matrix

[97] Amazon. (n.d.). 12V DC solenoid valve. Retrieved October 7, 2025

[98] eBay. (n.d.). 12V DC solenoid valve. Retrieved October 7, 2025, from https://ebay.us/m/cn3GSk

[99] Amazon, "DEVMO BTS7960B 43A Double DC Stepper Motor Driver Module H-Bridge PWM Smart Car Compatible with Arduino, Raspberry Pi," Amazon.com, [Online]. Available: https://www.amazon.com/DEVMO-BTS7960B-Stepper-H-Bridge-Arduino/dp/B07SZ4T699?adgrpid=177218412183&hvpone=&hvptwo=&hvadid=748029726649&hvpos=&hvnetw=g&hvrand=15004738281654529711&hvqmt=&hvdev=c&hvdvcmdl=&hvlocint=&hvlocphy=1013462&hvtargid=dsa-2418061332904&hydadcr=&mcid=&hvocijid=15004738281654529711--&hvexpln=67&tag=googhydr-20&hvsb=Computers_d&hvcampaign=dsadesk. Accessed: Oct. 7, 2025

[100] Wikipedia Contributors, “Braking distance,” Wikipedia, Oct. 30, 2019. https://en.wikipedia.org/wiki/Braking_distance

[101] HydraForce. (n.d.). HydraForce 12 volt DC NC solenoid valve SV10-22-0-N-00-9-5112-NC. Retrieved October 7, 2025, from https://www.surpluscenter.com/Brands/Hydraforce/Hydraforce-12-Volt-DC-NC-Solenoid-Valve-SV10-22-0-N-00-9-5112-NC.axd

[102] U.S. Solid. (n.d.). High pressure solenoid valve - 3/4” 12V DC brass 50 bar, 300°F high-pressure high-temperature solenoid valve. Retrieved October 7, 2025, from https://ussolid.com/products/high-pressure-solenoid-valve-3-4-12v-dc-brass-50-bar-300-high-pressure-high-temperature-solenoid-valve-html

[103] Adafruit Industries, “Raspberry Pi Model 3 A+,” Adafruit.com, 2020. https://www.adafruit.com/product/4027?srsltid=AfmBOorHuoPpt0qwaTCfBDaQneeeOvUpoC3Mu2bjvwnWXNr9S-t4B_VT (accessed Oct. 10, 2025).

[104] “Rotary actuators - heavy duty High Torque Gear Motors,” Firgelli Automations, https://www.firgelliauto.com/products/rotary-actuators (accessed Oct. 6, 2025).

[105] “Introduction To Electric Control Unit (ECU): Overview, Types,Components, Communication Protocols And Applications | DIYguru,” DIYguru, Feb. 03, 2025. https://diyguru.org/term/introduction-to-electric-control-unit-ecu/

[106] “Amazon.com: Generic CPC Offroad 200ML Orbital Steering Valve kit with Column and Plate : Automotive,” Amazon.com, 2025. https://www.amazon.com/Generic-Offroad-Orbital-Steering-Column/dp/B0CRCVWKCX (accessed Oct. 10, 2025).

[107] M. Maestro, “Micro Maestro 6-channel USB,” ServoCity®, 2025. https://www.servocity.com/micro-maestro-6-channel-usb/ (accessed Oct. 07, 2025).

[108] “EPS Steering DC Motor 48V 350W for Forklift Truck,” Made-in-China.com, 2025. https://lifttoppart.en.made-in-china.com/product/ZdLGBFeOwUVt/China-EPS-Steering-DC-Motor-48V-350W-for-Forklift-Truck.html (accessed Oct. 10, 2025).

[109] “Reverse Gearbox Kit for Go-Karts with TAV2 Series 30 Torque Converters,” Monster Scooter Parts, 2024. https://monsterscooterparts.com/products/reverse-gearbox-kit-for-go-karts-with-tav2-series-30-torque-converters?srsltid=AfmBOorvc4ThEiKghnmFOR8CgLV_MGSFAYSfSQa2ABHuyGoKIb5yRxij (accessed Oct. 06, 2025).

[110] “Ac Servo Motor,” indiamart.com, 2017. https://www.indiamart.com/proddetail/ac-servo-motor-10634229855.html (accessed Oct. 07, 2025).

[111] “ACS712 Current Sensor- 30A,” elecrow.openhardware, Oct. 24, 2013. https://www.elecrow.com/acs712-current-sensor-30a-p-710.html (accessed Oct. 10, 2025).

[112] G. Markel, “Power Steering Repair,” Brake & Front End, May 2005. https://www.brakeandfrontend.com/power-steering-repair/ (accessed Oct. 10, 2025).

[113] Nana and Nana, “Potentiometer for EPS pelletizer machine,” EPSTEC China EPS machine, Nov. 28, 2023. https://www.epstec.com/potentiometer-for-eps-pelletizer-machine/ (accessed Oct. 10, 2025).

[114] Kit, “Flaming River (FR40200KT): Microsteer Electric Power Steering Kit with Speed Sensor,” Parts Pro News, Apr. 29, 2021. https://partspro.com/blog/flaming-river-fr40200kt-microsteer-electric-power-steering-kit-with-speed-sensor/ (accessed Oct. 07, 2025).

[115] “Full Hydraulic Steering Kit, Most Toyota Truck 4WD PSC Performance Steering Components,” Vividracing.com, 2025. https://www.vividracing.com/full-hydraulic-steering-kit-most-toyota-truck-4wd-psc-performance-steering-components-p-152504776.html (accessed Oct. 07, 2025).

[116] “Actuonix P16-100-256-12-P,” Micro Linear Actuators & Servos - Actuonix Motion Devices, https://www.actuonix.com/p16-100-256-12-p?srsltid=AfmBOoqFERFEDu6hiy8GhymjCY623kLes-Jy3ms5LSeyBhe3DIMiicDL (accessed Oct. 6, 2025).

[117] NEMA 23 Stepper Motor 2.4nm(339.87oz.in) 4A 57x57x82mm 8mm shaft 4 wires - 23hs32-4004s | stepperonline, https://www.omc-stepperonline.com/nema-23-stepper-motor-2-4nm-339-79oz-in-4a-57x57x82mm-8mm-shaft-4-wires-23hs32-4004s (accessed Oct. 6, 2025).

[118] “L298N Motor Control Card Dual H-Bridge Module for DC and Stepper Mo...,” Fruugo, 2025. https://www.fruugo.us/l298n-motor-control-card-dual-h-bridge-module-for-dc-and-stepper-motors-2a-output-compatible-with-arduino/p-387982838-832641600?language=en&ac=google&utm_source=organic_shopping&utm_medium=organic (accessed Oct. 10, 2025).

[119] “Arduino Nano 33 BLE Rev2,” Arduino Online Shop, 2024. https://store-usa.arduino.cc/products/nano-33-ble-rev2?srsltid=AfmBOoqNUel3JqbeXyPycUX_-Ts1sSfs0c50039kNV9bGq75JhG89HrF (accessed Oct. 10, 2025).

[120] “LMC13 series,” P3 America, Inc., https://p3america.com/lmc13-series/?gad_source=1&gad_campaignid=11909418426&gbraid=0AAAAAD_cc-9VsiXAIGU2oTiCopftXqv7L&gclid=CjwKCAjwi4PHBhA-EiwAnjTHuY0I3WLJEMYY5LJTvj7RKkna2yPC5KaE7k0_ejk0cz2QrWyjSsBNTRoCmMIQAvD_BwE

[121] “Rotary Encoder - 1024 P/R (Quadrature),” Sparkfun.com, 2023. https://www.sparkfun.com/rotary-encoder-1024-p-r-quadrature.html

[122] Ha, “Royitay Kill Switch for 50cc 70cc 90cc 110cc 125cc 150cc 200cc 250cc ATV Dirt Bike Go Kart Pit Bike 4 Wheeler Quad Trail Bike Ha,” Carkart, 2025. https://carkart.com/products/royitay-kill-switch-for-50cc-70cc-90cc-110cc-125cc-150cc-200cc-250cc-atv-dirt-bike-go-kart-pit-bike-4-wheeler-quad-trail-bike-ha?srsltid=AfmBOop5i9r-PhPLPQBQkJbFkVE1WV1a0MGT6_8XeXEjF60Xo9-zGRWG (accessed Oct. 10, 2025)..

[123] “Welcome To Zscaler Directory Authentication,” Actuonix.com, 2025. https://www.actuonix.com/limit_kit?srsltid=AfmBOoqXyqdP-kfQCTFLkfzyWskN67jnNwFlqkgXJhllUCd7hhTUr7q4 (accessed Oct. 10, 2025).

[124] “PHATMOTO® Rover Kill Switch,” Phatmoto, 2025. https://www.phatmoto.com/products/phatmoto-rover-kill-switch (accessed Oct. 10, 2025).

## Appendix D: Gantt Chart for Current and Following Semester

Full Gantt Chart for Fall Semester

Full Gantt Chart for Spring Semester